# Feature Status

Factual status only -- no rankings. Maintain this file whenever something changes; it is the source of truth the README links to.

Status values: COMPLETE (works for its intended purpose today) - PARTIAL - SOFTWARE ONLY (works in software; never run on hardware) - HARDWARE REQUIRED (needs the physical machine to build/verify) - NEXT - FUTURE - DEFERRED (intentionally postponed until the machine is proven).

Horizon values: NOW (current scope, see CURRENT_SCOPE.md) - NEXT - FUTURE.

"SOFTWARE ONLY" means a software self-test or offline evaluation exists. It is never evidence of hardware success. Nothing in this repository has been validated on the physical machine except a read-only PLC link (2026-10-04).

| Feature | Current status | Now/Next/Future | Notes |
| --- | --- | --- | --- |
| Project management | COMPLETE | NOW | Multi-project folders, switch/create in the GUI. Not a job/recipe system |
| Dataset management | PARTIAL | NOW | Stage 1 images/labels managed in the GUI; Stage 2 dataset built by scripts in stage2_dataset/. Label tab: keyboard toggle, one-image multi-label inspector, balance strip, trash + label_log.csv + undo, import of a pre-sorted folder (new product or current), AI pre-labels (suggestions only, never training data until accepted). Software self-tested only; the AI pre-label path was tested with fake suggestions, not run against a real checkpoint in the self-test. No users/roles: the log records the OS login |
| Classification | SOFTWARE ONLY | NOW | Stage 1 multi-label classifier; held-out test results for 7 checkpoints. missing_cap cannot be learned from the current data: the 12 user images are one white-background bottle, and the 2026-10-05 candidate trained on them flagged every white-background bottle (shortcut; REJECTED, images removed, undoable). Needs missing-cap bottles photographed in the classifier's own setup |
| Detection | PARTIAL | NOW | YOLOv8n components (bottle/cap/label), not defects; opt-in runtime and the line's detection task. Every trained detector misses full-bottle missing caps (0/12: tamper ring read as cap, ~0.70) because that scene was test-only. v3 split puts it in train (stage2_dataset/split_v3.py); v3 candidate trained 2026-10-05: val missing cap 5/6, good 54/54 val + 50/50 test, learns the full-bottle bare neck (12/12 on its own training frames, not evidence; v1 active gets 0/12). CANDIDATE until validated on the machine |
| Segmentation | PARTIAL (interface only) | NOW | segment.py runtime interface (YoloSegmenter, label area / fill measurements) and decision rules (label_area_low, label_damaged) exist and are FAKE-tested. No segmentation model is trained: selecting the Segmentation task refuses to start the line with that reason; an inspection that needs it is FAULT |
| Annotation Studio | COMPLETE | NOW | Boxes + polygons, review flag, YOLO detection/segmentation export |
| YOLO training | COMPLETE | NOW | YOLOv8n only: 53 epochs, best epoch 33; YOLOv8s not trained |
| YOLO runtime | PARTIAL (SOFTWARE ONLY) | NOW | Current development. detect.py + Camera hook + Live-tab selector; boxes/confidence/detector state shown; observational (never PASS/REJECT); detector fault or stale -> FAULT. Live-tested only without a bottle in view; dev confidence 0.25 is not validated |
| Camera manager | PARTIAL | NOW | Discovery with device names, requested mode + delivered resolution/FPS/frame age, delivered-vs-requested + CAMERA_BANDWIDTH_PROBLEM, camera_controls lock, rotate, CAMERA TEST with image quality. Auto-reconnect while the line runs (background reopen every 3 s, bottles in the gap FAULT, attempts counted and logged). FAKE-tested; not tried on the EMEETs. No hardware trigger |
| Multi-camera | PARTIAL (2x EMEET Nova 4K tested live) | NOW | Both EMEETs stream together at 640x480 (~30 fps each). Only one streams at 1080p/720p: both are on one USB 2.0 hub (VID_1A40 PID_0101), measured 2026-10-03; needs separate USB ports. Per-bottle fusion FAULT > REJECT > PASS in decision.py. Cameras are not hardware-synchronised |
| Camera synchronization | SOFTWARE ONLY (time-based) | NOW | No hardware sync. Staggered stations (camera_stations, tracking.py): each camera's frames are taken in its own window (trigger + offset / speed), bounded by the next bottle, CAMERA_ASSOCIATION_FAULT when neighbours cannot be separated. FAKE-tested; never run on the belt |
| Trigger sensor | SOFTWARE ONLY | NOW | X0 -> M2 trigger drives the line (machine_cycle); X0 edges without a trigger are counted (BOTTLE_UNTRIGGERED). Physical photo-eye untested |
| Bottle tracking | SOFTWARE ONLY | NOW | Sensor-triggered association: each M2 trigger = one inspection_id, frames used are the ones captured strictly after the trigger. No visual tracking |
| Inspection window | SOFTWARE ONLY | NOW | inspect_frames per camera within inspect_window_s after the trigger (+ capture_delay_s); frame vote in decision.py. Values not tuned on the machine |
| Temporal voting | SOFTWARE ONLY | NOW | Frame vote per camera (decision.py majority / any / all) over the frames of one bottle |
| Decision engine | SOFTWARE ONLY | NOW | decision.py: one decision layer for classification / detection (per-project inspection recipe, config.json inspection: anchor + required parts + zones; default = bottle/cap/label: missing/misplaced cap, missing label, no bottle -> FAULT) / segmentation; frame vote; camera fusion; every failure -> FAULT. Thresholds are development defaults. No fault latching |
| PASS / REJECT / FAULT | SOFTWARE ONLY | NOW | Fail-safe states with freshness checks; software-tested with fake captures |
| Machine state | SOFTWARE ONLY | NOW | machine_state.py: one state (OFFLINE ... COMMUNICATION_FAULT) + start checklist; START enabled only when READY |
| Traceability | SOFTWARE ONLY | NOW | Every bottle persisted with run (job, product, recipe hash, model ids, SIMULATOR/REAL), decision, defects, PLC outcome, timings, per-camera windows, evidence path. inspection_trace.TraceStore (in-memory) still unused by the line |
| PLC communication | SOFTWARE ONLY (simulator) | NOW | plc/: Modbus ASCII client, PLCService (single owner, M2 trigger events, at-most-once PASS/REJECT, REJECT ack = M2 cleared because the ladder holds M1, X0 bottle accounting). FAKE-PLC tests pass against a scan emulation of the 09:06 ladder; simulator reads/faults verified; 09:06 ladder not yet run on the simulator (simulator had no program). Physical PLC untested. See PLC_COMMUNICATION.md 0.1 / K |
| Ladder simulation check | SOFTWARE ONLY | NOW | plc/ladder_sim.py + GUI Simulation check: the user's real .isp run in a scan simulator vs 14 software-contract scenarios (current ladder: structure OK, T0/T1 presets FAIL, masking/one-bottle LIMIT, no E-stop/interlock/timeout WARN); selfcheck.py runs every self-test. Does not read OR branches; not a hardware test |
| Stable operator verdict | SOFTWARE ONLY | NOW | verdict.py: one GOOD / DEFECT: / CHECKING / NO BOTTLE / FAULT per bottle, decided over several frames and latched until the bottle leaves; engineer view with boxes and score bars behind "Engineer details" |
| Scrolling pages | SOFTWARE ONLY | NOW | every page scrolls up/down and left/right (ScrollHost) |
| Database page and export | SOFTWARE ONLY | NOW | hmi.DatabaseTab, production_export.py; CSV and printable report; docs/guides/DATABASE.md |
| Threshold calibration | SOFTWARE ONLY | NOW | calibrate_thresholds.py (--apply / --restore); does not fix the good-bottle false alarms (72 good images) |
| Mock PLC | SOFTWARE ONLY | NOW | plc.test_simulation.FakePLC + FakeLadder: Modbus server + scan-by-scan emulation of the decoded ladder (nets 1-7, presets configurable), used by every PLC / machine-cycle / GUI self-test |
| Delta PLC | HARDWARE REQUIRED | NOW | First read-only serial link 2026-10-04 (USER-STATED). plc.ladder_check decodes the saved final_year.isp (unchanged since 2026-10-03 20:24, 7 networks) and checks 13 requirements: trigger / handshake / reject cycle / conveyor PRESENT; T0/T1 MISMATCH (15 s / 5 s); M10/M11, E-stop input, Y0 interlock, answer timeout, heartbeat ABSENT; triggers masked during a reject. See docs/hardware/PLC_LADDER_REQUIREMENTS.md. What is in the real PLC's memory is not yet compared |
| Conveyor | HARDWARE REQUIRED | NOW | Y1 read and shown (never written). Speed not measured: conveyor_mm_s = 0 until it is |
| Reject actuator | HARDWARE REQUIRED | NOW | Y0 read and tracked per REJECT bottle (never written; the PLC times it). Cylinder never fired on hardware |
| Timing engine | SOFTWARE ONLY | NOW | Time-based (no encoder): tracking.TimePositionSource, speed calibration wizard (>= 3 runs, spread limit, timestamped), line layout dialog, line_problems() blocks Start (T0 >= travel, unmeasured T0 > 5 s, downstream camera without speed, decision after REJECT dispatch). REJECT at trigger + travel - T0; late REJECT never fired. No distance or speed measured on the machine yet |
| Production counters | SOFTWARE ONLY | NOW | Production tab: Total / PASS / REJECT / FAULT / queue / not inspected / missed rejects; per-bottle row in projects/<slug>/production/<date>.csv |
| Evidence capture | SOFTWARE ONLY | NOW | production_store.py: first frame + overlay per bottle by evidence_policy (default REJECT_AND_FAULT), async writer, disk check; path in the bottle record |
| Job / recipe | SOFTWARE ONLY | NOW | Job + product per run; recipe editor dialog (engineer) for config.json inspection (anchor + parts + search/zone), validated against the detector classes, refused while the line runs; recipe hash stored per run |
| Model registry | SOFTWARE ONLY | NOW | model_registry.py + Models page: CANDIDATE / VALIDATED / APPROVED / ACTIVE / ARCHIVED / REJECTED, gated activation, deployments.jsonl, rollback, critical-class warning. Training no longer auto-activates |
| Dataset versioning | DEFERRED | FUTURE | Today: labels.csv copy per checkpoint; schema_version in Stage 2 JSON; hashes in MODEL_PROVENANCE.json |
| Traditional vision | DEFERRED | FUTURE | Only Sobel (ROI calibration) and Laplacian (sharpness) are used |
| Geometry measurement | FUTURE | FUTURE | No measurement in the code; shape defects are learned, not measured |
| Lighting configuration | HARDWARE REQUIRED | FUTURE | Not represented in software |
| Auto annotation / Annotation Studio | SOFTWARE ONLY | NOW | 2026-10-14. Backend vision/autoannotate.py + vision/annotate.py: per-proposal ACCEPT / REJECT / CORRECT (model's original kept) / SKIP, rejected proposals kept (hidden, undoable), per-image states (pending / accepted / corrected / rejected / failed / skipped), model id + confidence on every box, valid empty results distinguished from failures, invalid detections (NaN / out-of-range / outside image) dropped and counted, bulk accept / reject scoped to PENDING proposals by class / confidence / model, atomic save, audit log annotation_log.jsonl, export skips images with undecided proposals + export_manifest.json. Page ui/annotation_studio.py (3 zones: dataset + filters + queues + paged list; zoom / pan canvas with overlay toggles; inspector with Accept / Reject / Edit / Skip / Undo, bulk, history, export). Generation runs on one background thread in saved batches with progress, pause, cancel, model / class-mapping checks, refused while the line runs. Opens om_bottle's detector dataset (3,225 pending proposals incl. 389 bootstrapped defect boxes) although the project trains a classifier; labels.csv is never touched. Tested with fakes (python -m ui.annotation_studio, python -m vision.autoannotate); the real detector was not run through the page in a test. Polygon review states and a trained segmenter are not part of this |
| Auto-collect + hard examples | SOFTWARE ONLY | NOW | Live "Auto-collect for labelling": one frame per decided bottle into the inbox with the model's suggestion; corrections stored as hard examples (dataset.record_corrections); training oversamples good bottles (x5 cap) and hard examples (x3) (train.sample_weights). Not yet run on the machine |
| CHECK band (unsure bottles) | SOFTWARE ONLY | NOW | screen: CHECK: ? (check_margin 0.10); line: decision_rules.check_margin (off by default) -> FAULT |
| Activation gates | SOFTWARE ONLY | NOW | model_checks.py: real-camera limits + background-shortcut check enforced by model_registry.validate; Models -> Validate form |
| Active learning | SOFTWARE ONLY | NOW | Annotate tab queues: least sure, missing part (bottle without cap/label = likely real defect captures), doubtful part (cap/label at 0.25-0.80 = hard-negative candidates). Reorders unreviewed images only |
| Anomaly detection | DEFERRED | FUTURE |  |
| OCR | DEFERRED | FUTURE |  |
| Barcode | DEFERRED | FUTURE |  |
| SQLite | SOFTWARE ONLY | NOW | projects/<slug>/production/production.db: runs, bottles, alarms (stdlib sqlite3). FAKE-tested |
| Dashboard | SOFTWARE ONLY | NOW | Production page is the operator HMI (light theme, one machine state, both cameras, current bottle, counters, alarms, START / STOP / RESET FAULT, CAMERA TEST, TEST INSPECTION); Health page; operator / engineer modes |
| System report / specification check | SOFTWARE ONLY | NOW | 2026-10-14: system_report.bat (python -m tools.system_report [--plc] [--port COMx] [--no-cameras] [--no-models]) MEASURES computer (Python, torch/CUDA, GPU, RAM, disk), COM ports, saved ladder T0/T1 + R1-R13, line settings (labelled SAVED / MEASURED / NOT MEASURED, derived travel and reject-cycle times), cameras alone and together (bench.survey), model latency, data counts, and an optional READ-ONLY PLC link test through PLCService; compares each with SPEC in the tool -> reports/system/<time>/system_report.md/.json. First run 2026-10-14 (no PLC): EMEETs not connected, COM10 is Bluetooth, fault_action PASS, belt speed has no calibration record |
| Branding / design system | COMPLETE | NOW | 2026-10-14: product name NexaInspect from one file (app_assets/brand/brand.json, read by ui/brand.py); SVG masters + rendered PNG / ICO (python -m tools.brand_assets --build, build time only, no network); header mark + wordmark, window, dialog and taskbar icon (own AppUserModelID), launcher shortcut with the logo (python -m tools.make_shortcut [--desktop], not committed), Settings -> ABOUT with version 0.9.0 (1.0.0 reserved for the physically proven release). ui/theme.py light palette = NexaInspect tokens, navy navigation rail; FAULT stays amber (distinct from REJECT). Splash screen and logo on exported reports not done yet. Self-check: python -m ui.brand |
| User login | DEFERRED | FUTURE |  |
| Reports | SOFTWARE ONLY | NOW | History page: per day or per shift (A/B/C, configurable, night shift across midnight): counts, yield, defect distribution, bottles/hour, alarms, CSV export |
| Alarm management | SOFTWARE ONLY | NOW | alarms.py: 25 coded alarms (severity, message, action), condition / event, RESET FAULT acknowledges, persisted, shown on Production / History / Health |
| Maintenance | PARTIAL | NOW | Health page (computer / PLC / cameras / models / storage / alarms) + structured event logs (applog.py, 7 channels, searchable on Health) |
| Remote monitoring | DEFERRED | FUTURE |  |
| Closed-loop retraining | DEFERRED | FUTURE |  |

2026-10-19 - Camera rig commissioning: two cameras mounted diagonally on separate USB ports, autofocus off, LED lighting installed (USER-STATED, not yet measured by the software). Delivered 1080p50 and identical camera settings still to be verified; dataset capture in the real enclosure not started. See hardware/CAMERA_COMMISSIONING_CHECKLIST.docx.

## 2026-10-23 - San_test ladder timing model and timing check

- PLC ladder in use: plc file/San_test/San_test.isp (settings plc_ladder = san_test). M1 starts Y0 at once for T0 (K3 = 0.3 s); T1 is the heartbeat-lost timer (K30 = 3 s). Status: software + simulator tested (ladder_sim S1-S17, FakeLadder san_test, machine_cycle timing asserts); NOT run on the physical machine.

- timing_check.bat / tools.timing_check: plan (no hardware), apply, watch (read-only PLC), stopwatch travel. See guides/TIMING_CHECK_GUIDE.docx.

- Open: ladder has no rung that acts on M3; software halts on M3 through estop_device M3 (needs --apply). Travel repeatability is unmeasured.