Web Dashboard Development
Bottle Defect Detection System

Stage 1 – Universal AI Module | Stage 2 – Dynamic Machine Integration

# 1. Project Overview

The Web Dashboard is the Human Machine Interface (HMI) of the Bottle Defect Detection System. It acts as the central monitoring and management platform for the complete inspection process. The dashboard allows the administrator to manage datasets, train and evaluate AI models, switch between models and operating modes, benchmark camera and model performance, calibrate hardware, monitor live inspection, communicate with the PLC, and record production results.

# 2. Stage 1 – Universal Module (AI Development & Model Management)

Stage 1 focuses on creating a universal AI development environment. The objective is to build an architecture that can accept positive and negative datasets, train custom detection models, test different trained models, measure their performance, and select the most suitable model for deployment.

## 2.1 Dataset Management (Admin)

The administrator can upload, organize, inspect, and manage datasets used for training and evaluation.

Positive Dataset (Good Bottles):

Correct cap

Correct label

Correct bottle shape

Negative Dataset (Defective Bottles):

Missing cap

Tilted cap

Missing label

Damaged label

Skewed label

Broken/damaged bottle

Missing bottle

Other identified defects

Dashboard functions:

Upload images

Delete or manage images

Preview images

Display dataset statistics

Display class distribution

Configure train/validation/test split

Maintain dataset versions

## 2.2 AI Model Training

The dashboard provides an interface for training custom YOLO-based models. The administrator can select a model size and configure training parameters.

Model selection: YOLOv8n, YOLOv8s, YOLOv8m

Epochs

Batch size

Image resolution

Learning rate

Optimizer

Data augmentation settings

Training outputs to be stored/displayed:

best.pt

last.pt

Confusion matrix

Loss graphs

Precision graph

Recall graph

F1-score

Training logs

## 2.3 AI Model Management

Multiple trained models can be stored and managed from the dashboard.

Bottle detection model

Cap detection model

Label detection model

Combined inspection model

Model management functions:

Activate/deactivate a model

Rename a model

Delete a model

Download/export a model

Compare models

Select the model for live deployment

## 2.4 Operating Mode Switching

The dashboard should support multiple operating modes.

Training Mode

Upload dataset

Configure training

Train the model

Export/store the trained model

Testing Mode

Test saved images

Test recorded videos

Test stored datasets

View detection and performance results

Live Inspection Mode

Display live camera feed

Display detection bounding boxes/segmentation results

Display confidence score

Display PASS/FAIL decision

Display PLC communication status

Display bottle count and reject count

## 2.5 Model Performance & Accuracy Evaluation

The dashboard should evaluate both model accuracy and inference performance.

Accuracy metrics:

Precision

Recall

mAP@50

mAP@50–95

F1-score

Inference/system metrics:

Average inference time

FPS

CPU utilization

GPU utilization

Memory usage

Detection statistics:

Good/accepted bottles

Rejected bottles

False positives

False negatives

Detection confidence

## 2.6 Real-Time Performance Monitoring

During live operation, the dashboard continuously monitors the performance of the AI inspection pipeline.

Current FPS

Processing latency

Inference time

Frame drops

Detection confidence

Bottle throughput

PLC communication delay

Camera connection status

CPU/GPU utilization

Temperature monitoring if supported

## 2.7 Camera Benchmarking

For the project camera, the dashboard should benchmark the effect of camera resolution, frame rate, exposure, and image quality on inspection performance. The project uses an EMEET NOVA 4K webcam with a CMOS sensor.

Camera FPS

Actual captured FPS

Frame latency

Image brightness

Exposure

Sharpness

Motion blur

Resolution comparison

Lighting effect

Example camera comparison:

1920×1080 at 15 FPS

1920×1080 at 20 FPS

1920×1080 at 30 FPS

The selected camera configuration should be compared using detection accuracy, inference speed, CPU usage, memory usage, and overall inspection reliability.

## 2.8 AI Model Benchmarking

Different trained models can be benchmarked to select the best model for industrial deployment.

| Parameter | YOLOv8n | YOLOv8s | YOLOv8m | Purpose |
| --- | --- | --- | --- | --- |
| Precision | Record | Record | Record | Accuracy |
| Recall | Record | Record | Record | Defect coverage |
| FPS | Record | Record | Record | Real-time capability |
| Model Size | Record | Record | Record | Deployment resource |
| Inference Time | Record | Record | Record | Processing latency |

The actual values should be filled using measured results from the trained project models rather than assumed values.

# 3. Stage 2 – Dynamic Module (Machine Operation & Control)

Stage 2 focuses on integrating the AI detection system with the physical inspection machine. It converts the validated AI system into a real-time industrial inspection and control workflow involving the camera, sensor, conveyor, PLC, and pneumatic rejection mechanism.

## 3.1 Hardware Calibration

Hardware calibration should be completed before automatic inspection.

Conveyor calibration:

Conveyor speed

Bottle spacing

Encoder settings, if added

Trigger delay

Bottle travel/rejection timing

Camera calibration:

Lens distortion

Perspective

Intrinsic parameters

Extrinsic parameters

Camera position and inspection region

A checkerboard calibration target can be used to estimate camera calibration parameters. The calibration data should be saved and loaded by the inspection application.

Lighting calibration:

Brightness

Exposure

White balance

Gain

Contrast

Gamma

Uniform illumination across the inspection area

The project uses Wipro 6500 K LED lighting for the inspection enclosure. Lighting settings should be validated to minimize shadows, reflections, and variations in bottle appearance.

## 3.2 Detection Controller

The Detection Controller is the central runtime logic connecting image acquisition, AI detection, decision making, and physical rejection.

Bottle enters inspection area

Photoelectric sensor detects bottle

Camera is triggered

Image is captured

YOLO detection/segmentation is performed

Decision Engine evaluates the detected defects

PASS/FAIL result is generated

Result is communicated to the PLC

PLC activates or skips the pneumatic reject mechanism

Bottle is accepted or rejected

Inspection result is stored in the database

## 3.3 Controller Dashboard

The dashboard should provide live status of all major machine components.

Conveyor:

Running

Stopped

Emergency-stop state

Speed

Camera:

Connected/disconnected

FPS

Resolution

PLC:

Connected/disconnected

Communication status

Modbus status

Memory bits

Sensor status

Cylinder/solenoid status

AI:

Active model

Confidence score

Inference time

Detection FPS

## 3.4 Detection Settings

Authorized users should be able to configure the inspection parameters required for reliable detection.

Confidence threshold

NMS threshold

Minimum bottle size

Maximum bottle size

ROI (Region of Interest)

Reference line position

Reject delay

Reject distance

## 3.5 PLC Controller Integration

The dashboard communicates with the PLC through the selected industrial communication interface. For the project, the Delta DVP-series PLC can be integrated using Modbus communication over an appropriate RS-232/RS-485 interface or another supported communication method.

Dashboard/PLC functions:

PLC connection status

Read/write PLC memory bits

Read sensor states

Monitor solenoid valve status

Monitor conveyor motor status

Trigger the reject actuator

Monitor emergency-stop status

Manual/automatic mode selection

The exact electrical interface and communication protocol should match the specific Delta PLC model installed in the prototype.

## 3.6 Live Production Dashboard

During production, the dashboard should display:

Total production count

Accepted bottles

Rejected bottles

Current bottle status

Detected defect type

Detection confidence

Cycle time

Bottles per minute

Shift-wise production

Hourly production trend

## 3.7 Database & Inspection Reports

Every inspection can be stored for traceability, quality analysis, and future model improvement.

Recommended inspection log fields:

Timestamp

Inspection image

Detection result

Defect class

Confidence score

PLC response

Conveyor speed

Camera settings

Active AI model

Reports can be generated by:

Hour

Shift

Day

Week

Month

Export options:

CSV

PDF

Excel

# 4. Overall System Architecture

The complete dashboard architecture can be represented as the following logical flow:

Admin Web Dashboard
        │
        ├── Dataset Manager
        ├── Model Manager
        └── System Settings
                 │
                 ▼
        AI Training Engine
                 │
                 ▼
        Trained YOLO Models (.pt)
                 │
                 ▼
        Detection Engine
                 │
                 ▼
        Live Camera (EMEET 4K)
                 │
                 ▼
        Photoelectric Sensor Trigger
                 │
                 ▼
        Decision Engine (PASS / FAIL)
                 │
        ┌────────┴────────┐
        ▼                 ▼
    Delta PLC          Database
        │                 │
        ├── Conveyor      ├── Images
        ├── Rejector      ├── Logs
        └── Sensors       └── Reports
        │                 │
        └────────┬────────┘
                 ▼
      Live Production Dashboard

# 5. Stage-wise Project Objective

Stage 1 – Universal:

Develop a reusable AI model management and training architecture.

Support positive and negative datasets for bottle quality inspection.

Provide model training, testing, switching, and benchmarking.

Evaluate accuracy and real-time inference performance.

Benchmark camera configurations for reliable inspection.

Select and deploy the best-performing model.

Stage 2 – Dynamic:

Integrate the AI system with the physical inspection hardware.

Calibrate the camera, lighting, conveyor, and inspection parameters.

Implement real-time bottle detection and PASS/FAIL decision logic.

Communicate inspection decisions to the PLC.

Control the pneumatic rejection mechanism through the PLC.

Monitor production, defects, machine status, and system performance.

Maintain inspection logs and generate quality reports.

# 6. Final Project Concept

The final web dashboard will function as a centralized Industrial AI inspection interface. Stage 1 provides the universal AI development, model management, accuracy evaluation, and benchmarking environment, while Stage 2 connects the selected AI model to the actual machine and enables dynamic calibration, detection, PLC control, pneumatic rejection, production monitoring, and traceability. This creates a complete Camera → Computer Vision → Python/Decision Engine → PLC → Actuator workflow suitable for the Bottle Defect Detection System prototype.