# Problem Statement and Objectives

## Problem statement

Manual visual inspection of bottles on a quality-control line for defects such as damage, skew, tilted caps, incorrect fill level, and label defects is labour-intensive and prone to inconsistency. An automated, camera-based inspection system that classifies defects and issues a PASS/FAIL verdict per bottle can support a QC line, provided the underlying classifier is evaluated rigorously enough that its reported accuracy can be trusted on images it has never seen.

## Objectives — Stage 1 (completed)

- Build a labelled, multi-label image dataset of bottle images across eight candidate defect classes, organised by project, with a labels.csv file as the source of truth.

- Establish a data split that avoids leaking near-duplicate frames of the same physical bottle between training, validation, and test — the dataset consists of consecutive video frames of a small number of physical bottle passes ("scenes"), so a naive random split would overstate accuracy.

- Train a multi-label defect classifier and measure its performance honestly on a held-out test set that is never used for training, validation-time model selection, or threshold tuning.

- Compare more than one candidate backbone architecture under an identical protocol (same split, same batch size, learning rate, and test fraction) to select a defensible baseline architecture, rather than accepting a single untested model.

- Document, rather than hide, the limitations of the current dataset (thin classes, absent classes, no genuine multi-defect examples).

## Objectives — future stages (not yet started)

See 15_Future_Work/FUTURE_WORK.md. These are not part of Stage 1 and are listed here only to state the project's intended direction; none of them have been implemented or evaluated.

## Success criteria for Stage 1

- A working, reproducible training and evaluation pipeline (train.py, dataset.py, infer.py).

- At least one classifier architecture evaluated on a genuinely held-out test set, with per-class precision/recall/F1, confusion counts, and inference speed reported.

- A comparison across multiple architectures under the same protocol, producing a documented, evidence-based selection of a baseline architecture.

- Explicit documentation of dataset limitations that affect the confidence of the reported results (see 09_Limitations/DATASET_LIMITATIONS.md).

All of the above have been achieved and are documented in this workspace.