ABVIS - Automated Bottle Visual Inspection System

Final project review pack: review summary, presentation slides, demo script and viva preparation

[Students]  |  [Guide]  |  [Department, College]  |  2026-27

Right-click > Update field

# Part A - Final Review Summary

## A.1 One-paragraph summary

ABVIS is an AI machine-vision station that inspects 250 ml bottles on a conveyor and makes a PLC reject the bad ones. It combines an EfficientNet defect classifier, a YOLOv8 component detector, a voting decision engine and a Delta PLC handshake over Modbus. It also includes an operator HMI, a model registry and a production database. On held-out data the classifier scores macro-F1 0.949 (0 defective bottles passed) and the detector mAP50 0.979, and the control software passes 18/18 self-tests. A real trial and a real-camera test showed a camera domain gap, collapsed thresholds and a PLC throughput limit. All three are diagnosed, with a phased plan to industrial grade.

## A.2 Objectives vs achievement

Table 1: Objectives

| Objective | Status | Evidence |
| --- | --- | --- |
| Leak-free multi-label dataset | Done (limited size) | 1167 reviewed images, ~106 scenes, scene split |
| Compare classifiers, select on test | Done | 5 architectures; EfficientNet-B0 selected |
| Component detector | Done | YOLOv8s v3 test mAP50 0.979 |
| Decision engine (vote, fusion, FAULT) | Done | self-tests |
| PLC integration and reject timing | Done in simulation; read-only on the real PLC; trial run | ladder simulator S1-S14, trial |
| HMI, alarms, registry, database | Done | 15 pages, SQLite record of 200 bottles |
| Validated on the real line | Not yet | trial: 0 PASS / 4 REJECT / 196 FAULT |

## A.3 Key numbers

Table 2: Key numbers

| Metric | Value |
| --- | --- |
| Classifier test macro-F1 (EfficientNet-B0, leak-free) | 0.949 |
| Classifier false PASS / false REJECT (226 test bottles) | 0 / 9 |
| Classifier inference | 8.4 ms/image (RTX 3050) |
| Detector test mAP50 / mAP50-95 (YOLOv8s v3) | 0.979 / 0.818 |
| Detector inference | 10.9 ms/image |
| Software self-tests | 18 / 18 pass |
| Code base | ~28,000 lines Python, 7 packages |
| Real-camera frames called defective by classifiers | 33 / 33 (every model) |
| Real-line trial | 200 bottles: 0 PASS, 4 REJECT, 196 FAULT (167 not inspected) |

## A.4 Contributions

A complete, safety-first inspection architecture: unknown is FAULT, no late rejects, a single PLC owner.

Honest evaluation: scene-based leak-free splits, a test set used once, real-camera and real-line evidence.

A model registry with activation gates and rollback.

PLC ladder decoding and scan simulation to test the PLC program against the software contract.

A diagnosis of why laboratory accuracy did not transfer, with a measurable upgrade plan.

# Part B - Presentation (slide by slide)

20 slides, about 15-18 minutes. Each slide lists its title, on-screen points, the figure to show and what to say.

## Slide 1: Title

ABVIS - Automated Bottle Visual Inspection System

AI defect detection + PLC-controlled rejection

[Students] - [Guide] - [Department, College]

Speaker notes: Introduce the team and the one-line idea: a camera inspects every bottle and a cylinder removes the bad ones.

## Slide 2: Problem

Manual inspection is slow, tiring and inconsistent

Defects: missing / tilted cap, damaged / skewed / missing label, damaged / skewed bottle, fill level

Goal: inspect every bottle, reject bad ones, never pass an uninspected bottle

Figure 1: Slide 2 visual

Speaker notes: Show the eight defect classes on real bottles.

## Slide 3: Objectives

Dataset with leak-free split

Classifier + detector

Decision engine

PLC integration and reject timing

HMI, registry, database

Validate on the real line

Speaker notes: These are the seven objectives; the review summary table shows which are done.

## Slide 4: System architecture

PC: HMI, cameras, AI, decision, machine cycle, PLC service, database

PLC: owns conveyor, T0 delay, T1 pulse, cylinder

Modbus ASCII over RS-232

Figure 2: Slide 4 visual

Speaker notes: Walk left to right: cameras > AI > decision > PLC > cylinder. Stress: the PC never writes an output.

## Slide 5: Machine cycle

X0 > M2 trigger > frames after trigger > AI > vote

PASS: M0 now

REJECT: M1 at trigger + travel - T0 > PLC fires Y0

Figure 3: Slide 5 visual

Speaker notes: Explain time-based tracking: travel 9.5 s from the measured distance and belt speed.

## Slide 6: Hardware

Delta DVP-SS2 PLC, photo-eye, Festo cylinder + 5/2 valve

2 x EMEET Nova 4K cameras

i7-12700H, RTX 3050 4 GB

Figure 4: Slide 6 visual

Speaker notes: Show the real station frames next to the training images.

## Slide 7: Dataset

1167 reviewed images, ~106 scenes

Scene-based split: no near-duplicates across sets

Detection: 594 images, 1,994 boxes

Figure 5: Slide 7 visual

Speaker notes: Explain why scenes, not images, are the real sample size.

## Slide 8: Classifier

EfficientNet-B0 + 4 other backbones, multi-label sigmoid

BCE + pos_weight, AdamW, cosine, 25 epochs

Weighted sampling for good bottles and hard examples

Figure 6: Slide 8 visual

Speaker notes: Point out that validation saturates at 1.0 early: the data, not the epochs, is the limit.

## Slide 9: Classifier results

Best: EfficientNet-B0, test macro-F1 0.949

0 defective bottles passed, 9 good rejected (226)

Weakest class: tilt_cap

Figure 7: Slide 9 visual

Speaker notes: Compare architectures; justify B0 by accuracy, zero escapes and speed.

## Slide 10: Per-class view

Most classes ≈ 1.0 F1

tilt_cap ≈ 0.73 for every model

missing_label: no test positives

Figure 8: Slide 10 visual

Speaker notes: A global score is weak for small angles; measurement is the fix.

## Slide 11: Detector

YOLOv8n / YOLOv8s on bottle, cap, label

YOLOv8s test mAP50 0.979

YOLOv8s still improving at epoch 80

Figure 9: Slide 11 visual

Speaker notes: Explain mAP50 and mAP50-95; label is the hardest class.

## Slide 12: Detector quality

Per-class precision / recall / mAP

PR curve and confusion matrix

Figure 10: Slide 12 visual

Speaker notes: Highlight the label mAP50-95 (0.62): box edges of a wrap-around label are ambiguous.

## Slide 13: Decision engine and safety

Recipe: required parts in zones

Majority vote over frames; FAULT > REJECT > PASS

Unknown is FAULT; FAULT rejected; late reject never fired; halt latch

Speaker notes: This is what keeps the line safe even when the AI is wrong.

## Slide 14: Software

15-page HMI, operator / engineer modes

Model registry: candidate > validated > approved > active

SQLite production record, alarms, logs

18 / 18 self-tests

Figure 11: Slide 14 visual

Speaker notes: Show a screenshot or the live demo.

## Slide 15: Real-camera test

Classifiers: 33 / 33 real frames called defective

Detector: bottles found, empty scenes clean, caps unreliable

Figure 12: Slide 15 visual

Speaker notes: The domain gap: lab accuracy did not transfer to the new camera.

## Slide 16: Real-line trial

200 bottles: 0 PASS, 4 REJECT, 196 FAULT

167 not inspected: one-bottle PLC handshake

Safety worked: everything uninspected recorded as FAULT

Figure 13: Slide 16 visual

Speaker notes: Present honestly: the failure is understood and measurable.

## Slide 17: Root causes

Domain gap + thresholds at 0.05

One-bottle handshake + long window

Timestamps not exact (15.6 ms clock, serial latency)

Figure 14: Slide 17 visual

Speaker notes: Walk through the time budget bar chart.

## Slide 18: Upgrade plan

Phase 0 safe settings

Phase 1 exact timing + three-zone confidence + real data

Phase 2 PLC FIFO + defect localisation

Phase 3 industrial cameras, encoder

Phase 4 measurement, validation

Figure 15: Slide 18 visual

Speaker notes: Explain the three-zone decision: confident defect / confident good / unsure > reject.

## Slide 19: Conclusion

Complete, safety-first inspection stack built and tested

Strong held-out results

Real-world gaps found with evidence and a plan to close them

Speaker notes: End on what was learned: real validation decides, not lab scores.

## Slide 20: Questions

Thank you

Speaker notes: Have backup slides ready: dataset split, PLC I/O map, timing formula.

# Part C - Live Demo Script (no hardware needed)

Launch: python gui.py. The app opens in OPERATOR mode on the Production page.

Switch to ENGINEER mode (header button). Show the page groups.

Machine page: connect to the ISPSoft simulator (TCP 127.0.0.1:10002), or explain the real COM link.

Production > engineer row > Simulation check: run 'Check the LADDER' (14 scenarios) and 'Check the SOFTWARE' (self-tests).

Production > TEST INSPECTION on a saved image: show the verdict and the overlay.

Label page: show labels.csv-backed labelling and the inbox. Annotate page: show boxes and proposals.

Models page: show the CANDIDATE > ACTIVE lifecycle and gates.

Database page: open the trial records, export the CSV report.

Terminal backup: python selfcheck.py (18/18 pass in about 2 minutes).

# Part D - Viva Questions and Answers

Q: Why multi-label instead of multi-class?

A: Defects can co-occur (a skewed bottle with a damaged label). One sigmoid per defect allows any combination; softmax would force exactly one.

Q: Why split by scene?

A: Images are consecutive video frames. A random split puts near-duplicates of one bottle in train and test and inflates accuracy. Whole scenes go to one split only.

Q: Why did you choose EfficientNet-B0?

A: Highest held-out macro-F1 (0.949), zero defective bottles passed, and 8.4 ms per image.

Q: What is macro-F1 and why not accuracy?

A: The mean of per-class F1. Accuracy is misleading under imbalance: always answering 'no defect' scores high accuracy while missing every defect.

Q: Why both a classifier and a detector?

A: The classifier judges appearance (damage, skew, fill). The detector judges presence and position of parts (a missing cap is a bottle without a cap box).

Q: What is mAP50?

A: Mean average precision when a predicted box counts as correct at IoU ≥ 0.5 with the true box; mAP50-95 averages IoU thresholds 0.5 to 0.95 and rewards tight boxes.

Q: How is the reject timed without an encoder?

A: Travel time = distance / measured belt speed. The PC sends M1 at trigger + travel - T0; the PLC waits T0 and fires the cylinder exactly when the bottle arrives.

Q: What happens if the camera or model fails?

A: The bottle becomes FAULT and is rejected by default; three FAULTs in a row halt the line; an alarm explains why.

Q: Why does the PC never write PLC outputs?

A: Outputs are owned by the deterministic PLC. The PC only answers PASS or REJECT, so a PC crash cannot fire or block the cylinder directly.

Q: Why did the classifier fail on the real camera?

A: Domain gap: different camera, background, framing and crop; and thresholds at 0.05 that fire on any unfamiliar image.

Q: Why were 78 % of bottles uninspected?

A: The ladder accepts a new trigger only after the previous bottle is answered; the window and reject cycle made each bottle occupy the PLC for 4-12 s while bottles arrive every ~0.9 s.

Q: Would more epochs help?

A: Not for the classifier: validation was already perfect at epoch ~12, so it would only memorise. Yes for YOLOv8s, which was still improving at epoch 80, but after adding production-camera data.

Q: How will you reach high confidence?

A: Calibrate scores (temperature scaling), use two limits per defect (confident defect / confident good) and reject the unsure middle; gate production on escape rate ≤ 0.1 %.

Q: How accurate are your timestamps?

A: Consistent but not exact: the Windows monotonic clock in Python 3.8 ticks every 15.6 ms, frames are stamped after read() returns, and the trigger after the serial reply. The plan switches to perf_counter, brackets each stamp and measures latency with an LED test.

Q: What is the role of the model registry?

A: No trained model reaches production until it passes a held-out test, a real-camera check and a shortcut check, and is approved; every activation is logged and reversible.

Q: How do you add a new product?

A: Images + labels, a detector trained on its parts, and a recipe in config.json; no code changes.

# Part E - Checklist before the review

Fill names, roll numbers, guide and college on the title pages.

Add application screenshots (Production, Label, Models, Database) to the black book.

Set fault_action to REJECT before any live demo with the PLC.

Print the results summary table and the upgrade-plan table as handouts.

Rehearse the real-trial slides: present them as findings, not failures.