# ABVIS: Automated Bottle Visual Inspection System

A camera looks at every bottle on a conveyor. A defective bottle is pushed off the line by a pneumatic cylinder. A bottle the system cannot vouch for is treated the same way. An uninspected bottle must never pass as good.

First product: 250 ml Bisleri water bottles. Defects: missing cap, tilted cap, damaged label, skewed label, missing label, damaged bottle, skewed bottle, wrong water level. Everything runs in one Windows desktop application: labelling, training, the model registry, live inspection, PLC communication and the production record.

Status on 2026-10-13 (read this first).

- The whole machine cycle exists in software and passes its self-tests against a fake PLC, fake cameras and fake models. These are software tests, not hardware tests.

- On the real line (2026-10-06/07, 200 bottles in production.db), 0 bottles passed, 4 were rejected and 196 were FAULT. 167 of those were never inspected because the PLC's one-bottle handshake was still busy.

- On the production EMEET cameras the detector returns boxes that cover almost the whole frame. Neither model has been validated on the production camera.

- The configured classifier (20260919-164511) and detector (stage2_best.pt) had been moved to the archive; they were copied back on 2026-10-13 and load again.

- settings.json still has "fault_action": "PASS". Since 2026-10-13 START refuses it (except in MONITOR ONLY): set FAULT -> REJECT on the Production page.

- Industrial upgrade, first round (2026-10-13): one high-resolution clock, bracketed trigger timestamps, AI off the reject-deadline thread, three-zone confidence decision with an UNSURE count, defect localisation (where the defect is, in the record and on the evidence image), calibrated training, settings audit trail. Details: INDUSTRIAL_UPGRADE_PLAN.md - Implementation status.

The full audit and the upgrade plan: INDUSTRIAL_UPGRADE_PLAN.md.

## Contents

- How the system works

- Architecture

- The machine cycle, step by step

- Camera layer and timestamps

- The AI models

- Datasets

- Decision engine and safety rules

- PLC integration

- The application (15 pages)

- Model lifecycle and folders

- Installation and running

- Configuration reference

- Tests

- Repository map

- Known issues

- Glossary and further docs

## 1. How the system works

flowchart LR
    A[Bottle on conveyor] --> B[Photo-eye X0]
    B --> C[PLC sets M2: inspect now]
    C --> D[PC takes frames AFTER the trigger]
    D --> E[Classifier + Detector]
    E --> F{Decision engine}
    F -->|GOOD| G[M0: pass]
    F -->|DEFECT| H[M1 at travel - T0]
    F -->|FAULT| H
    H --> I[PLC times T0, fires Y0 for T1]
    F --> J[(SQLite record + evidence)]

- A photo-eye (X0) sees the bottle. The PLC ladder sets M2: "inspect now".

- The PC's PLC service sees M2 rise and creates a trigger with a timestamp.

- Each line camera has a frame window (trigger + camera offset / belt speed). Only frames captured strictly after the window opens are used.

- The classifier scores each frame for every defect. The detector finds bottle / cap / label boxes; the recipe turns a missing or misplaced part into a finding.

- The decision engine votes the findings across frames and fuses cameras: FAULT > REJECT > PASS.

- PASS: the PC writes M0 at once. REJECT/FAULT: the PC writes M1 at trigger + travel - T0, and the PLC itself times T0 and pulses Y0 (the cylinder) for T1. The PC never switches an output.

- Every bottle becomes one row in projects/<slug>/production/production.db, with evidence images by policy.

Golden rule: unknown is never good. A dead camera, a crashed model, a stale frame or a bottle nobody saw becomes FAULT, and FAULT is rejected (when fault_action is REJECT, the default).

## 2. Architecture

┌──────────────────────────── Windows PC (Python 3.8, one process) ────────────────────────────┐
│                                                                                               │
│  ui/app.py + ui/hmi.py (CustomTkinter, Tk main thread, App.q drained every 60 ms)             │
│     │                                                                                         │
│     ├── vision/infer.py  Camera ×N ── grab thread (stamps frames) ── score thread (live view) │
│     │                                                                                         │
│     ├── line/machine_cycle.py  MachineCycle (one deadline thread)                             │
│     │      ├── Inspector: frame windows → classifier / detector / segmenter → decision.py     │
│     │      ├── FIFO of bottles, each with its own REJECT deadline                             │
│     │      ├── line/tracking.py (time-based position, camera stations, association)           │
│     │      ├── line/alarms.py, line/production_store.py (SQLite), line/applog.py (logs/)      │
│     │      └── PLCService ────────────────────────────────────────────┐                       │
│     │                                                                  │                       │
│     └── registry/model_registry.py  CANDIDATE → VALIDATED → APPROVED → ACTIVE → ARCHIVED      │
│                                                                        │                       │
└────────────────────────────────────────────────────────────────────────┼───────────────────────┘
                                                                         │ Modbus ASCII
                                              RS-232 COMx 9600 7E1 ──────┤ (or TCP 127.0.0.1:10002
                                                                         │  to the ISPSoft simulator)
                                                        Delta DVP-SS2 PLC (ladder owns Y0/Y1, T0, T1)

Key design rules (enforced in code, see CLAUDE.md):

- One owner of the PLC link (plc/service.py). Python writes only M0 (PASS) and M1 (REJECT), plus opt-in commissioning bits. It never writes X or Y.

- One answer per trigger, never retried. The trigger is marked answered before the write.

- Every module reads project paths from vision/dataset.py globals at call time. Switching project is D.use_project(name).

- labels.csv is the source of truth. Folders only seed labels when an image is first imported.

- One crop path (dataset.model_input) for training and inference.

- Tk widgets are touched only on the main thread. Background work reports through App.q.

## 3. The machine cycle, step by step

| Step | Code | What happens |
| --- | --- | --- |
| 1. Trigger | plc/service.py::_observe | M2 rising edge → Trigger(id, seen_mono, seen_wall). With watch_x0, an X0 rise without an M2 rise is BOTTLE_UNTRIGGERED → a FAULT "NOT INSPECTED" bottle. |
| 2. New bottle | machine_cycle._on_trigger | Bottle gets an id, scheduled_mono = trigger + travel. If halted → FAULT "NOT ANSWERED". M2 already ON at start → cleared with M0 and not inspected. |
| 3. Collect | Inspector.begin / poll | Per camera: window [trigger + offset + delay, + window_s]. Frames with start < ts < next bottle's start, spread one per slot. Each frame is scored as it arrives. |
| 4. Decide | Inspector.evaluate → decision.decide | Per-frame findings → majority vote → camera fusion. |
| 5. Command | MachineCycle._decided / _send / _due | PASS → M0 now. REJECT → M1 at scheduled - T0. Late by more than reject_tolerance_s → not fired; answered M0, recorded FAULT, alarm "remove by hand". |
| 6. Observe | _drain_events | Y0 ON/OFF observed → bottle finished. No Y0 → REJECT_NOT_OBSERVED. |
| 7. Record | _finish → ProductionStore.record | One SQLite row per bottle; evidence image on a background writer. |
| 8. Safety latch | halt() | STOP, E-stop input, fault_latch_after consecutive FAULTs (default 3) or a crash. While halted no M0/M1 is sent. |

Timing (no encoder):

travel time = inspection_to_reject_mm / conveyor_mm_s        (or the stopwatch value reject_travel_s)
M1 sent at  = trigger + travel - T0
Y0 fires at = trigger + travel

timing_problem() / fit_problem() block START when the verdict cannot be ready before travel - T0.

## 4. Camera layer and timestamps

vision/infer.py::Camera runs two threads per camera:

- Grab thread (_grab_loop): cap.read() in a loop (DirectShow, MJPG 1920×1080 requested). Right after read() returns, the frame is stamped t_grab = time.monotonic(), rotated if configured, given seq = frame_seq + 1, and pushed to recent (a deque of the last 12 frames, ~0.4 s at 30 fps).

- Score thread (_infer_loop): scores the newest frame at ≤ 15 Hz for the Live page only. The production line does not use it: Inspector scores the frames of each bottle itself.

Freshness rules: a score or frame older than MAX_RESULT_AGE_S = 1.0 s is FAULT. (session, seq) never repeats across restarts. Wall-clock time is never used for freshness.

What the timestamp really means (verified 2026-10-13). The stamp is "when read() returned", not "when the sensor exposed the image". On this PC's Python 3.8, time.monotonic() is GetTickCount64 with 15.6 ms resolution; time.perf_counter() is QPC with 100 ns resolution. The PLC trigger is stamped after the serial read reply arrives. Both effects, and the fixes, are in INDUSTRIAL_UPGRADE_PLAN.md §3.

## 5. The AI models

|  | Stage 1: Classifier | Stage 2: Detector | Stage 3: Segmenter |
| --- | --- | --- | --- |
| Code | vision/train.py, vision/infer.py | vision/detect.py, vision/yolo_stage2_train.py, registry/model_bench.py | vision/segment.py, stage2_dataset/seg_pipeline.py |
| Model | EfficientNet-B0 (also B1, ResNet18, MobileNetV3-S, ConvNeXt-T) | YOLOv8n / YOLOv8s (Ultralytics 8.1.0) | interface only, no weights |
| Input | Fixed ROI crop, letterboxed and centre-cropped to 192×448 | Full frame, 640 | Full frame |
| Output | One sigmoid per defect (multi-label) | Boxes: bottle, cap, label (components, not defects) | Label polygon |
| Loss / training | BCE + pos_weight, AdamW, cosine LR, 25 epochs, balanced sampler | 80 epochs, patience 20, mosaic, workers=0 | n/a |

### Current checkpoints (classifier, project om_bottle)

| Stamp | Arch | Held-out test macro-F1 | False pass | False reject (of 226) | Weakest class |
| --- | --- | --- | --- | --- | --- |
| 20261008-081732 | EfficientNet-B0 | 0.949 | 0 | 9 | tilt_cap F1 0.73 |
| 20261008-090538 | ConvNeXt-T | 0.920 | 0 | 19 | tilt_cap 0.73, damaged_label 0.81 |
| 20261008-083918 | MobileNetV3-S | 0.902 | 10 | 10 | skewed_label 0.70 |
| 20261008-082510 | ResNet18 | 0.877 | 0 | 10 | skewed_label 0.73 |
| 20261008-083257 | EfficientNet-B1 | 0.821 | 18 | 14 | tilt_cap 0.06 |
| 20260919-164511 (archived, still set as active_model) | EfficientNet-B0 | 0.956 | 0 | 10 | tilt_cap 0.81 |

Validation macro-F1 reaches 1.000 by epoch ~12 on every run. The validation set is too easy to choose an epoch or a threshold with. The 2026-10-08 runs include the 22 white-background missing-cap images, which earlier caused a "white background = missing cap" shortcut (see §15).

### Current detector candidates (models/stage2_yolo/)

| Candidate | Epochs run / best | Val mAP50 / 50-95 | Test mAP50 / 50-95 | Label recall (test) |
| --- | --- | --- | --- | --- |
| bench_yolov8s_v3 | 80 / 70 (still improving) | 0.984 / 0.859 | 0.979 / 0.818 | 0.990 |
| bench_yolov8n_v3 | 48 / 28 (early stop) | 0.982 / 0.853 | see candidate_yolov8n_v3.json |  |

These scores come from the 594-image Stage 2 dataset. On the production EMEET camera the old stage2_best.pt produced near-full-frame boxes (captures/TEST_*, captures/CHECK_*).

## 6. Datasets

| Dataset | Location | Size | Used for |
| --- | --- | --- | --- |
| Classification | projects/om_bottle/images/{+ve,-ve/<Class>,_inbox} + labels.csv | 1,202 images (1,167 reviewed), ~106 scenes | Stage 1 |
| Detection | stage2_dataset/ (annotations_v2.json, split_v3.json, yolo_export_v3/) | 594 images, 1,994 boxes, 3 classes | Stage 2 |
| Unified manifest | vision_dataset/manifests/unified_manifest.csv | 1,749 rows | leak-free splits, audits |
| Raw source | All Datasets/ (+ All Datasets.zip, 1 GB) | same images as om_bottle | archive |

Classification class balance (reviewed images / distinct scenes):

| damaged_bottle | damaged_label | skewed_bottle | tilt_cap | water_level | good | skewed_label | missing_label | missing_cap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 319 / 25 | 229 / 21 | 167 / 17 | 145 / 18 | 112 / 12 | 72 / 8 | 71 / 8 | 30 / 2 | 22 / white bg |

All classification images are 2537×1927 and no image carries more than one defect. The production cameras deliver 1920×1080, rotated 90°. Splits are by scene, never random: the images are consecutive video frames.

## 7. Decision engine and safety rules

line/decision.py is pure functions (no I/O):

- Classification: hits = [d for d in probs if probs[d] >= threshold[d]]. A NaN or empty score is FAULT.

- Detection: gate_detections keeps one bottle (aspect ≥ 1.3, area ≥ 2 %, nearest station_x) and its parts by per-class confidence. The recipe (config.json → inspection; default bottle / cap / label) turns "required part not found in its zone" into missing_cap / missing_label, and a part outside its zone into cap_misplaced.

- Vote: majority across the frames in which a bottle was found. No bottle in any frame is FAULT.

- Fusion: FAULT > REJECT > PASS across cameras.

- Optional unsure band: check_margin > 0 makes a score just below its threshold a FAULT ("check"). Default 0.

Safety rules the code never breaks: unknown is FAULT; the AI never commands the PLC directly; one answer per trigger; a late REJECT is never fired; the real PLC never auto-connects; the hardware E-stop must cut power by itself. The software HALT latch only stops the program from answering.

## 8. PLC integration

- plc/protocol.py: Modbus ASCII frames, LRC. plc/address_map.py: the only device map and write policy.

- plc/client.py: PLCClient + TcpTransport (simulator) / SerialTransport (real PLC, pyserial).

- plc/service.py: PLCService, one worker thread, triggers, events, the M0/M1 handshake.

- plc/ladder_check.py: decodes plc file/final_year/final_year.isp and checks 13 requirements (read-only).

- plc/ladder_sim.py: runs the decoded ladder in a scan simulator against scenarios S1-S14.

- plc/test_simulation.py::FakeLadder: scan-by-scan emulation used by every machine-cycle self-test.

Real link: COM port, RS-232, ASCII 9600 7E1, station 1. Delta COMMGR holds the COM port while its driver exists, so close it first. The saved ladder has T0 K150 / T1 K50 (15 s / 5 s); plc_t0_s / plc_t1_s must equal what is really in the PLC. Ladder gaps (no E-stop input, no answer timeout, triggers masked during a reject, one bottle at a time): docs/hardware/PLC_LADDER_REQUIREMENTS.md.

## 9. The application (15 pages)

| Group | Pages | Who |
| --- | --- | --- |
| PRODUCTION | Production, History, Database, Health | operator |
| DATA | Label, Defects, Annotate, Data health | engineer |
| MODEL | Train, Analysis, Models (registry) | engineer |
| ENGINEERING | Live, Machine (PLC), Camera (benchmark) | engineer |
| SYSTEM | Settings | engineer |

Operator mode shows only PRODUCTION. Engineer mode (header button, optional PIN) shows everything plus the engineer row on Production: AI task, line cameras, timing, speed calibration, camera stations, recipe, simulation check, HALT latch, simulator bottle feed.

Machine states (line/machine_state.py): OFFLINE → NOT_READY → READY → INITIALIZING → RUNNING ⇄ INSPECTING → STOPPING, plus FAULT, E_STOP and COMMUNICATION_FAULT. START is enabled only in READY.

## 10. Model lifecycle and folders

flowchart LR
    T[train / model_bench] --> C[CANDIDATE]
    C --> V[VALIDATED<br/>held-out test + real-camera note + shortcut check]
    V --> A[APPROVED]
    A --> AC[ACTIVE]
    AC -->|replaced| AR[ARCHIVED]
    AR -. rollback .-> AC

Training never activates a model (unless auto_activate_trained_model is true). Activation writes config.json active_model + thresholds (classifier) or settings.json detector_weights (detector, sha256 checked against MODEL_PROVENANCE.json). Gates (registry/model_checks.py): good bottles called defective ≤ 2 %, defective bottles passed ≤ 1 %, at least 30 good and 30 defective real-camera bottles.

| Folder | What is in it |
| --- | --- |
| projects/om_bottle/models/<stamp>/ | Current classifier candidates: model.pt, metrics.json, test_metrics.json, labels.csv |
| models/stage2_yolo/ | Current detector candidates (bench_yolov8{n,s}_v3) |
| models_archive/previous/ | Every model before the 2026-10-08 retrain, including the configured active_model and stage2_best.pt |
| FINAL_YEAR_BLACKBOOK/19_Retrain_2026-10-08/ | Metrics and logs of that retrain, with the previous results |

The archive naming convention for future retrains is in INDUSTRIAL_UPGRADE_PLAN.md §8.

## 11. Installation and running

# Windows 11, Python 3.8 (keep from __future__ import annotations; no 3.9+ syntax)
pip install torch torchvision --index-url https://download.pytorch.org/whl/cu124   # or /cpu
pip install -r requirements.txt
pip install ultralytics==8.1.0 pyserial comtypes psutil

python -m vision.calibrate     # once per product: measure the crop ROI
python gui.py                  # the application

run.bat does the same on double-click. Known problems with run.bat today: its ROI check runs import dataset, which no longer exists after the package move, so it re-runs vision.calibrate on every launch. It also tries tools.migrate --run whenever an All Datasets/ folder exists (it refuses, but prints an error). Fixes: plan Phase 0.

First run: the app opens in OPERATOR mode. Switch to ENGINEER → Machine → Simulator or Real PLC → Connect → Production → choose cameras, check timing → START when the banner says READY. Without hardware use CAMERA TEST, TEST INSPECTION or Simulation check.

Command line:

python -m vision.train --epochs 25 --arch efficientnet_b0          # a CANDIDATE
python -m registry.model_bench cls-test <stamp>                      # held-out test, once
python -m registry.model_bench yolo --model yolov8s.pt --data v3 --workers 0
python -m registry.model_bench det-defects --weights <pt> --tag <t> --split-version v3
python -m vision.calibrate_thresholds [--apply | --restore]

Always pass workers= to Ultralytics on Windows: the default 8 workers exhausts a 16 GB machine.

## 12. Configuration reference

settings.json (per machine, not in Git; a running app rewrites it, so stop the app before editing):

| Key | Current | Meaning | Production value |
| --- | --- | --- | --- |
| fault_action | PASS | What a FAULT bottle gets | REJECT |
| plc_mode / plc_com | serial / COM10 | Link type / port (real PLC was on COM5) | measured |
| plc_t0_s / plc_t1_s | 5.0 / 2.5 | Must equal the ladder's T0 / T1 | ladder values |
| inspection_to_reject_mm / conveyor_mm_s | 1028 / 108.2 | Travel = 9.5 s | measured |
| inspect_frames / inspect_window_s | 13 / 3.9 | Frames per camera / watch window | see plan |
| line_cameras / camera_controls | [2, 3] / rotate 90 | Line cameras, locked driver controls | lock focus/exposure/WB |
| threshold_shift / threshold_overrides | -0.3 / {...} | Live tuning on top of config.json thresholds | remove |
| decision_rules.det_min_conf_by_class | bottle 0.2, cap 0.2, label 0.4 | Detector gates (code default 0.55 / 0.40 / 0.40) | see plan |
| detector_weights / detector_conf | not set / default 0.25 | Detector checkpoint / confidence | set on activation |
| plc_operator_controls | true | Commissioning bits M10/M11/M2 writable | false |
| evidence_policy | default REJECT_AND_FAULT | Which bottles keep an image |  |

projects/<slug>/config.json: roi, roi_frame, input_wh (192×448), cache_wh, thresholds, active_model, inspection (recipe). The model's own ROI, baked into its checkpoint, overrides roi at inference.

## 13. Tests

Every module is its own self-check (fakes only; none is a hardware test):

python selfcheck.py            # quick set (~2 min)
python selfcheck.py --full     # + dataset, training maths, inference, detector, whole GUI
python -m line.machine_cycle   # full cycle against FakeLadder + fake cameras
python -m plc.ladder_sim       # the real ladder vs the software contract
python gui.py --selftest       # builds every tab with a fake PLC

Known stale self-test: stage2_dataset/review_app.py --selftest fails by design (the review is finished). Tests must never write into real projects/*/production, logs/ or settings.json.

## 14. Repository map

gui.py, selfcheck.py, run.bat         launchers
settings.json                         per-machine settings (not in Git)
vision/   dataset, train, infer (Camera, Model), detect (YOLO), segment, calibrate, calibrate_thresholds,
          bench, verdict, vision_data, annotate, autoannotate, yolo_stage2_train
line/     decision, machine_cycle, machine_state, tracking, alarms, production_store, production_export,
          inspection_trace, applog
registry/ model_registry, model_bench, model_checks
ui/       app (main window, ~6k lines), hmi, theme, charts, annotation_studio
tools/    selfcheck, migrate, camera_planner
plc/      protocol, address_map, client, service, commissioning, handshake_test, ladder_check, ladder_sim, tests
plc file/final_year/                  the user's ISPSoft ladder (read-only; never edited by this repo)
projects/<slug>/                      images, labels.csv, config.json, models/, production/, cache/, thumbs/
stage2_dataset/                       detection dataset: annotations, splits v1/v2/v3, export + validation scripts
vision_dataset/                       unified manifest and leak-free classification export
models/stage2_yolo/                   current detector candidates
models_archive/previous/              models before the 2026-10-08 retrain
captures/                             bench / TEST INSPECTION captures with result.json
logs/                                 app, camera, ai, plc, machine, alarm, production (rotating)
docs/                                 guides, hardware, roadmap, audits, design
FINAL_YEAR_BLACKBOOK/                 academic write-up and results

## 15. Known issues

Ranked; the fixes are phased in INDUSTRIAL_UPGRADE_PLAN.md.

| # | Severity | Issue |
| --- | --- | --- |
| 1 | Critical Mitigated | fault_action is PASS in settings; START now refuses it outside monitor-only |
| 2 | Critical Fixed | Configured models were archived; copied back 2026-10-13 |
| 3 | Critical | 78 % of real bottles were not inspected: one-bottle PLC handshake + 3.9 s window + 4.5 s reject dispatch |
| 4 | Critical | Models do not generalise to the production camera (near-full-frame boxes, undecided scores) |
| 5 | High | Classifier crop: training ROI from a 2537×1927 landscape frame is rescaled onto a 1080×1920 portrait frame |
| 6 | High (training fixed) | train.py now takes the middle of the best band + calibration; existing models still have 0.05 thresholds until retrained; live threshold_shift -0.3 remains |
| 7 | High Fixed | Three-zone rule (pass_max 0.10) and min_bottle_frames_frac 0.6 are on for the line |
| 8 | High Improved | Trigger bracketed (est + uncertainty); status read 1 Hz while running; still polled serial (hardware trigger is Phase 3) |
| 9 | High | Data: 72 good images (8 scenes), missing_label 2 scenes, no multi-defect images, different camera |
| 10 | High | 22 white-background missing-cap images are back in om_bottle (the known background shortcut) |
| 11 | Medium (clock fixed) | line/clock.py (perf_counter) replaces the 15.6 ms clock; driver latency still unmeasured (LED test, Phase 1.5) |
| 12 | Medium Fixed | AI runs on its own worker thread (Inspector.async_ai) |
| 13 | Medium Fixed | run.bat import fixed |
| 14 | Medium | yolo_export*/data.yaml points to a non-existent E:/MACHINE LEARNING PROJECT/Datasets/... path |
| 15 | Low Fixed | Duplicate definitions removed |
| 16 | Engine done | Recipe defect classes + locations; 389 defect boxes proposed for review (vision.defect_bootstrap); a v4 detector still has to be trained |
| 17 | High | New 2026-10-08 classifier 20261008-081732 scores missing_cap ~1.00 on every real frame (white-background shortcut): do not activate |

## 16. Glossary and further docs

| Term | Meaning |
| --- | --- |
| X0 / M2 / M0 / M1 / Y0 / Y1 / T0 / T1 | photo-eye / "inspect now" / PASS answer / REJECT answer / cylinder / conveyor / reject delay / reject pulse |
| Trigger | M2 rising edge seen by the PC |
| FIFO | Bottles between camera and cylinder, each with its own deadline |
| Recipe | What a complete product is: anchor (bottle) + required parts in zones |
| FAULT | "The system cannot vouch for this bottle" |
| Candidate | A trained model not yet approved for production |

| Document | Purpose |
| --- | --- |
| INDUSTRIAL_UPGRADE_PLAN.md | Audit of 2026-10-13 and the phased plan to commercial readiness |
| CLAUDE.md | Every rule the code relies on (developer notes) |
| docs/TEAM_HANDBOOK.md | Full system handbook |
| docs/roadmap/FEATURE_STATUS.md | Per-feature truth |
| docs/roadmap/PLC_COMMUNICATION.md | PLC contract, decoded ladder |
| docs/hardware/PLC_LADDER_REQUIREMENTS.md | Rungs the ladder still needs |
| docs/guides/PRODUCTION_HMI_AND_COMMISSIONING.md | Operator and commissioning guide |
| docs/guides/DATABASE.md | Production database |
| docs/README.md | Index of all docs |