# Team handbook: everything about the Bottle Inspection System

Who this is for: a new team member, a reviewer, or an AI assistant (ChatGPT, Claude, ...) who must understand the whole system without asking anyone. It is self-contained: hardware, technology, features, how it works, how to run and test it, every setting, current status, known problems and what to do next.

Status date: 2026-10-06. Everything is software-tested with fakes (fake PLC, fake cameras, fake models). The full inspect -> reject cycle has never run on the physical machine. The only physical contact so far is a read-only link to the real PLC (2026-10-04).

Related files: README.md (front page) - docs/PROJECT_BRIEF_FOR_REVIEW.md (status + review questions) - CLAUDE.md (every rule the code relies on) - docs/roadmap/FEATURE_STATUS.md (per-feature truth).

## Contents

- What the system does - 2. Hardware - 3. Technology -

- Architecture - 5. A bottle's journey - 6. The PLC -

- The AI - 8. Data and files - 9. The application -

- Settings reference - 11. Operations -

- Safety rules - 13. Status -

- Troubleshooting - 15. Next steps - 16. Glossary -

- Document map - 18. Using this with an AI assistant

## 1. What the system does

A camera inspects every bottle on a conveyor. Bad bottles, and bottles the system cannot judge, are pushed off the belt by a pneumatic cylinder. Unknown is never good. First product: 250 ml Bisleri water bottles. Defects: missing cap, tilted cap, damaged bottle, damaged label, missing label, skewed bottle, skewed label, wrong water level.

It is one Windows desktop application that does everything: label images, train the AI, run the line, talk to the PLC, show one answer per bottle, keep a production record, and manage models.

## 2. Hardware

| Item | Detail (mostly USER-STATED; almost nothing measured) |
| --- | --- |
| Conveyor | about 5 ft (1524 mm), bottle transit 14-16 s (roughly 95-110 mm/s); no encoder |
| Sensor | photoelectric bottle sensor -> PLC input X0 |
| PLC | Delta DVP-SS2, ISPSoft ladder, RS-232 Modbus ASCII 9600 7E1, COM5, station 1 |
| Actuator | Festo DSNU cylinder + 5/2 solenoid valve (PLC output Y0) at the reject station |
| Cameras | 2 x EMEET Nova 4K (USB). On one USB 2.0 hub only one 1080p stream works; use separate USB 3 ports |
| Lighting / enclosure | 2 LED strips 6500 K, black matte enclosure 450 x 320 x 400 mm |
| Planned layout | Camera 1 at the photo-eye on one wall; Camera 2 on the opposite wall, shifted toward the exit |
| Computer | Windows 11, Python 3.8, RTX 3050 Laptop GPU (4 GB), 16 GB RAM |

## 3. Technology

| Layer | Choice |
| --- | --- |
| Language / OS | Python 3.8 (keep from __future__ import annotations; no 3.9+ syntax at runtime), Windows 11 |
| UI | CustomTkinter 6.0 (+ plain Tk widgets for per-frame video), Pillow; light industrial theme, dark optional |
| Vision | OpenCV 4.13 (DirectShow cameras, crops, overlays) |
| AI | PyTorch 2.4.1 + CUDA 12.4, torchvision (EfficientNet-B0 and 4 other backbones), Ultralytics 8.1.0 (YOLOv8n/s) |
| PLC link | Modbus ASCII over serial (pyserial) or TCP (ISPSoft simulator, 127.0.0.1:10002); own protocol code in plc/ |
| Storage | CSV (labels.csv), JSON (configs, provenance), SQLite (production record), JPEG (evidence) |
| Charts | drawn on Tk canvas (no matplotlib) |
| Optional | psutil (CPU/RAM readings), comtypes (camera names), pynvml (GPU util) |
| Tests | each module is its own self-test; python selfcheck.py [--full] runs all 25 |

requirements.txt has the basics; ultralytics==8.1.0, pyserial, comtypes, psutil are installed separately. run.bat does setup + launch on Windows.

## 4. Architecture

flowchart TB
    subgraph UI[Desktop app: gui.py + hmi.py]
        P[Production] --- H[History / Database / Health]
        L[Live / Machine / Camera] --- M[Models / Train / Label / Annotate]
    end
    subgraph CORE[Machine path]
        MC[machine_cycle.py<br/>FIFO + deadlines] --> DEC[decision.py<br/>recipe, vote, fusion]
        MC --> TR[tracking.py<br/>time-based position]
        MC --> PLCS[plc/ PLCService<br/>single owner of the link]
        MC --> REC[production_store.py<br/>SQLite + evidence]
    end
    subgraph AI[Perception]
        CAM[infer.py Camera threads] --> CLS[classifier]
        CAM --> DET[detect.py YOLO]
        CAM --> VER[verdict.py<br/>one steady verdict]
    end
    UI --> CORE
    CORE --> AI
    PLCS --> PLC[(Delta PLC ladder)]
    PLC --> CYL[Reject cylinder]

Key design rules (each has a reason in CLAUDE.md):

- dataset.py module globals: use_project(name) rebinds every path; all modules read D.NAME at call time.

- One decision layer (decision.py); AI models never command the PLC.

- One owner of the PLC link (PLCService); application code never touches PLCClient.

- One machine state (machine_state.py) shown everywhere; one steady verdict per bottle (verdict.py) for screens.

- Tk is single-threaded: background work reports through App.run_bg / App.post.

- Configuration over code: product rules are a recipe (JSON); line timing and layout are settings.

### Module map

| Area | Files |
| --- | --- |
| Machine cycle, decision, tracking | machine_cycle.py, decision.py, tracking.py, machine_state.py, verdict.py |
| PLC | plc/protocol.py, address_map.py, client.py, service.py, test_simulation.py (FakePLC + FakeLadder), ladder_check.py, ladder_sim.py, commissioning.py |
| Perception | infer.py (cameras, classifier, overlays), detect.py (YOLO runtime), segment.py (interface, no model) |
| Training and models | train.py, model_bench.py, yolo_stage2_train.py, calibrate_thresholds.py, model_registry.py, model_checks.py |
| Data and labelling | dataset.py, annotate.py, autoannotate.py, annotation_studio.py, vision_data.py, calibrate.py, migrate.py |
| Record and alarms | production_store.py, production_export.py, alarms.py, applog.py, inspection_trace.py (older, in-memory) |
| App | gui.py (~5k lines), hmi.py, theme.py, charts.py, bench.py |
| Checks | selfcheck.py |
| Data on disk | projects/<slug>/, stage2_dataset/, models/, plc file/ |

## 5. A bottle's journey

X0 (photo-eye) -> PLC ladder sets M2 ("inspect now")
  -> computer takes frames AFTER the trigger from each camera (camera 2 at trigger + offset / belt speed)
  -> AI: classifier (defect scores) + YOLO (bottle / cap / label boxes)
  -> decision.decide(): votes over frames, fuses cameras, applies the recipe -> PASS / REJECT / FAULT
  -> PASS: write M0 now.  REJECT or FAULT: write M1 at (trigger + travel time - T0)
  -> the PLC runs T0, then drives Y0 (cylinder) for T1 by itself
  -> one row per bottle in SQLite + evidence picture + event logs

- travel time = distance(camera -> cylinder) / belt speed; both are measured settings (0 = not measured; then T0 is used).

- FAULT is rejected by default (fault_action). A REJECT that would be late is never fired; the bottle is FAULT and an alarm says to remove it by hand.

- A bottle that reaches X0 while the PLC is busy has no trigger: recorded NOT INSPECTED (alarm BOTTLE_UNTRIGGERED).

- Two cameras: frames are taken in each camera's own window; if two bottles are too close to tell apart by time the camera's evidence is refused (CAMERA_ASSOCIATION_FAULT) rather than judging the wrong bottle.

- The screens show GOOD / DEFECT:  / CHECKING / NO BOTTLE / FAULT (and CHECK: ? for unsure bottles); the PLC, database internals and code keep PASS / REJECT / FAULT.

## 6. The PLC

| Device | Meaning | Who writes |
| --- | --- | --- |
| X0 | photo-eye | hardware |
| X1 / X2 | start / stop buttons | hardware |
| M2 | "inspect now" trigger | PLC (rising edge of X0) |
| M0 | PASS answer | computer |
| M1 | REJECT answer | computer |
| Y0 | reject cylinder | PLC only |
| Y1 | conveyor motor | PLC only |
| T0 | delay (travel / lead) timer | PLC |
| T1 | reject pulse timer | PLC |
| C0 / C1 | PASS / REJECT counters | PLC |
| M10 / M11 | commissioning start/stop pulses | computer (opt-in plc_operator_controls) |

The ladder is the user's (plc file/final_year/final_year.isp): 7 networks, read and simulated by this project but never edited. Current ladder: structure correct; T0 = K150 (15 s), T1 = K50 (5 s) (the software expects about 1.5 s and 0.5 s); no E-stop input, no Y0/Y1 interlock, no answer timeout, no watchdog; a bottle during a reject cycle gets no trigger. Full list and the rungs to add: docs/hardware/PLC_LADDER_REQUIREMENTS.md. Check it any time: python -m plc.ladder_check (requirements) and python -m plc.ladder_sim (14 scenarios).

Command rules (never weaken): one answer per trigger; trigger marked answered before the write; no retries (an unknown outcome is reported as WRITE_FAILED / NOT_ACKED / ACK_LOST); preconditions re-read right before the write; success = the PLC clears the command bit and M2 (M1 is held through the reject cycle); a trigger seen again after a reconnect is a new trigger flagged after_reconnect; Python never writes X or Y devices.

Real PLC connection: close Delta COMMGR first (it keeps the COM port open); the real PLC is never connected automatically; physical test commands need engineer mode + test mode + an arm click.

## 7. The AI

### Models

|  | Classifier (Stage 1) | Detector (Stage 2) |
| --- | --- | --- |
| Model | EfficientNet-B0 (also B1, ResNet18, MobileNetV3, ConvNeXt), multi-label, 8 outputs | YOLOv8n (3 classes: bottle, cap, label) |
| Input | ROI crop of the bottle, 192 x 448 (tall, not square) | whole frame |
| Active | checkpoint 20260919-164511 | models/stage2_yolo/stage2_best.pt (v1) |
| Missing cap | cannot detect it (no usable positives) | a bottle with no cap box, via the recipe |

Recipe (config.json "inspection", editable in the app): anchor class + parts with required, search, zone. A new product needs data + detector + recipe, not code.

Verdict for screens (verdict.py): ignore the first frames, need 6 valid frames, a defect counts if it is in >= 60 % of them, then latch GOOD / DEFECT until the bottle has left; a score just under its threshold in most frames is CHECK; invalid frames -> FAULT, never GOOD. Without a detector (presence unknown) it uses a rolling window.

### Results so far (offline only)

| Measure | Value |
| --- | --- |
| Classifier held-out test (238 images) | macro-F1 0.935; old thresholds: 10 of 19 good bottles called defective, 0 defective passed |
| After threshold calibration (applied) | 22 false alarms instead of 28, but 1 of 219 defective passed; still 10 of 19 good flagged |
| Detector test mAP50 / mAP50-95 | v1 0.968 / 0.660, v2 0.979 / 0.793, v3 0.971 / 0.803 (v3 test = 52 images) |
| YOLOv8s (v1 data) | 0.973 / 0.676; v2 run unfinished (stopped at epoch 27/80); v3 not trained |
| Missing cap, detector on validation (6) | v1 0/6, v2 6/6, v3 5/6; the 12 full-bottle bare necks: v1 0/12 |

### Model lifecycle (nothing reaches production automatically)

Train -> CANDIDATE -> VALIDATED (held-out test + real-camera numbers: good called defective <= 2 %, defective passed <= 1 %, >= 30 + 30 real bottles, and for a classifier the background-shortcut check) -> APPROVED -> ACTIVE, with models/deployments.jsonl and rollback (model_registry.py, model_checks.py, Models page).

### Improvement loop

Live Auto-collect saves one frame per decided bottle (with the model's guess) into the Label inbox -> you review -> corrections become hard examples -> training draws good bottles up to x5 and hard examples x3 more -> new candidate. Details: docs/guides/AUTO_ANNOTATION.md.

### The open problem: missing caps

The 22 missing-cap pictures are one unlabelled bottle on a white background. Detectors read the green tamper ring as a cap; a classifier trained on them learned "white background = missing cap" (it flagged 25 of 25 capped white bottles) and was rejected. Keep those images out of classifier training. Needed: missing-cap bottles from several bottles/poses photographed in the real enclosure. Detector v3 (missing-cap scene in train) is a CANDIDATE that must be validated on the real machine.

## 8. Data and files

projects/<slug>/            one product: labels.csv, config.json, project.json  (tracked in Git)
  images/{+ve, -ve/<Class>, _inbox}   models/<stamp>/   production/production.db   cache/   trash/   (NOT in Git)
projects/active.txt         which project opens
stage2_dataset/             detection dataset provenance (annotations_v2.json, split.json, split_v3.json, ...); images NOT in Git
models/                     MODEL_REGISTRY.json, deployments.jsonl, stage2_yolo/ (weights NOT in Git)
logs/<channel>.log          app, camera, ai, plc, machine, alarm, production (NOT in Git)
settings.json               machine / UI settings (NOT in Git; per PC)
plc file/final_year/        the PLC project (read only)

- labels.csv is the source of truth (one row per image, one column per defect, reviewed flag). Folders only seed a label once; unreviewed images are never trained on; model suggestions are never labels until a person accepts.

- Scene split: near-duplicate video frames are grouped into scenes and kept whole in train / val / test; the test set is only read, never used to choose thresholds or epochs.

- om_bottle (classifier): 1,167 images, 8 defects, 72 good. Stage 2 (detector): 594 images, 39 scenes, 1,994 boxes (splits v1, v2 tight cap boxes, v3 missing-cap scene in train).

- Label edits are logged and reversible (label_log.csv, trash/, dataset.undo).

## 9. The application: pages and features

15 pages in five groups; every page scrolls up/down and left/right. OPERATOR mode (default) shows PRODUCTION only; ENGINEER mode (header button, optional PIN) shows everything.

| Group | Page | What it does |
| --- | --- | --- |
| PRODUCTION | Production | one machine state, start checklist, both cameras live, current bottle, counters, alarms; START INSPECTION / STOP / RESET FAULT; CAMERA TEST; TEST INSPECTION (no PLC). Engineer row: AI task, cameras, timing, Speed calibration, Line layout / camera stations, Recipe, Simulation check, HALT latch, simulator feed |
|  | History | per day or shift: counts, yield, defect distribution, bottles/hour, alarms, evidence, CSV |
|  | Database | bottles / alarms / runs tables, filters, plain-words columns, CSV for Excel, printable report |
|  | Health | computer, PLC, cameras, models, storage, alarms; searchable logs |
| DATA | Label | thumbnail grid, multi-label tagging, AI pre-labels, undo, import |
|  | Defects | add / rename / delete defect classes |
|  | Annotate | boxes and polygons; model proposals; PREDICTED: GOOD / DEFECT line; active-learning queues |
|  | Data health | class balance, ROI, duplicate / leak warnings |
| MODEL | Train / Analysis | train a CANDIDATE; curves, per-defect results |
|  | Models | registry: Validate form, Approve, ACTIVATE, Reject, Roll back |
| ENGINEERING | Live | tuning view; one verdict; Engineer details; Auto-collect; snapshot into a label; thresholds |
|  | Machine | PLC connection manager (simulator / real), live X/M/Y/T/C, armed test PASS/REJECT, Simulation check |
|  | Camera | measures what each camera mode really delivers; focus / exposure / white balance lock |
| SYSTEM | Settings | text size, theme, evidence policy, auto-activate (off), engineer PIN, Simulation check switch |

Engineer tools: Speed calibration (>= 3 timed runs, refused if spread > tolerance); Line layout (camera stations, distances, per-camera roles); Recipe editor; Simulation check (ladder simulation + all self-tests + live PLC read); Validate form (real-camera numbers + shortcut check).

Machine states: OFFLINE, NOT_READY, READY, INITIALIZING, RUNNING, INSPECTING, STOPPING, FAULT, E_STOP, COMMUNICATION_FAULT. START is enabled only in READY (PLC connected and in RUN, cameras chosen, models present, timing valid, E-stop released, recipe valid).

Alarms: 25 coded alarms (severity, message, recommended action), condition or event, RESET FAULT acknowledges.

Database: SQLite projects/<slug>/production/production.db: runs (each START: job, product, models, recipe fingerprint, SIMULATOR / REAL PLC), inspections (one row per bottle), alarms. Rows are only added. Guide: docs/guides/DATABASE.md. Evidence pictures follow evidence_policy.

## 10. Settings reference

settings.json (per PC; the running app rewrites some keys, so stop the app before editing). Defaults in brackets.

| Key | Meaning |
| --- | --- |
| plc_mode ["tcp"], plc_host, plc_port [127.0.0.1:10002], plc_com, plc_baud [9600], plc_format ["7E1"], plc_station [1] | PLC link (simulator TCP or real serial) |
| plc_operator_controls [false] | enable commissioning buttons (M10/M11 pulses, virtual bottle M2) |
| plc_t0_s, plc_t1_s | must equal the ladder's T0 / T1 (default 1.5 / 0.5 s) |
| inspection_to_reject_mm, conveyor_mm_s [0 = not measured] | travel time = distance / speed |
| speed_tolerance_pct [10], timing_margin_s [0.15], speed_calibration | tracking uncertainty and last calibration record |
| camera_stations | {"<source>": {name, role, offset_mm, side, rules: {station_x, judge}}} camera positions and roles |
| line_cameras, line_task, line_capture_wh [1920x1080], line_fourcc ["MJPG"] | which cameras / AI task / capture mode |
| inspect_frames [3], inspect_window_s [0.6], capture_delay_s [0] | frames per camera per bottle, collection window |
| reject_tolerance_s [0.3], fault_action ["REJECT"], fault_latch_after [3] | late-reject limit, what to do with FAULT, consecutive FAULTs that halt the line |
| estop_device ["" or "X3"], estop_active_high [false] | optional PLC input wired to the E-stop status |
| decision_rules | overrides for decision.RULES (vote, det_min_conf, check_margin, ...) |
| camera_controls | {"<index>": {focus, exposure, wb_temperature, rotate}} locks applied on every open |
| detector_conf [0.25], detector_weights | YOLO confidence (development value) and weights path |
| check_margin [0.10] | score within this margin below a threshold = unsure -> CHECK (0 = off) |
| evidence_policy ["REJECT_AND_FAULT"] | NONE / ALL / REJECT_ONLY / FAULT_ONLY / REJECT_AND_FAULT / SAMPLE:n |
| autocollect_daily_cap [300] | max auto-collected frames per day |
| train_balance_good [true], train_good_cap [5], train_hard_factor [3] | training sampling |
| auto_activate_trained_model [false] | keep false: a trained model must pass the registry |
| activation_gates | override limits (max_good_called_defective_pct 2, max_defective_passed_pct 1, min_real_good 30, min_real_defective 30, max_shortcut_fire_pct 10) |
| critical_classes [["missing_cap"]] | classes the registry flags if the test set has no positives |
| job_id, product | typed on the Production page, stored with every run |
| shifts | [["A","06:00","14:00"], ...] |
| ui_mode ["operator"], ui_theme ["light"], engineer_pin, font_scale [1.0], show_simulation_check [true] | UI |

Per-project config.json: roi, roi_frame, input_wh, cache_wh, thresholds, active_model, inspection (recipe), thresholds_before_calibration.

## 11. Operations: run, test, train, deploy

pip install torch torchvision --index-url https://download.pytorch.org/whl/cu124
pip install -r requirements.txt && pip install ultralytics==8.1.0 pyserial comtypes psutil
python -m vision.calibrate            # first run: measure the classifier ROI
python gui.py                  # the application

Test: python selfcheck.py (quick, ~2 min) or --full (25 checks incl. the GUI). python -m plc.ladder_sim checks your ladder. When the GPU/RAM is busy: set CUDA_VISIBLE_DEVICES= first.

Train and deploy a classifier: python -m vision.train --epochs 25 --arch efficientnet_b0 (CANDIDATE) -> python -m registry.model_bench cls-test <stamp> (held-out test, once) -> python -m vision.calibrate_thresholds [--apply|--restore] -> Models page: Validate (real-camera numbers + shortcut check) -> Approve -> ACTIVATE (line stopped); Roll back if needed.

Train a detector: python -m registry.model_bench yolo --model yolov8n.pt --data v3 --workers 0 (always pass workers; the default exhausts memory on 16 GB), then python -m registry.model_bench det-defects --weights <pt> --tag <t> --split-version v3.

First physical run (order): connect PLC read-only -> read T0/T1 in the PLC and fix presets -> photo-eye test -> cameras on separate USB 3 ports, CAMERA TEST -> Speed calibration -> Line layout -> TEST INSPECTION on known good / defective bottles -> armed PASS test -> armed REJECT test (cylinder guarded) -> one bottle at a time -> sequential cameras -> multi-bottle -> latency -> E-stop. Full 16 steps: docs/guides/PRODUCTION_HMI_AND_COMMISSIONING.md.

Git: work on a branch, open a pull request, merge. Weights, images, settings.json, logs/ are never committed.

## 12. Safety rules (never weaken)

- Unknown is FAULT, never PASS; a FAULT bottle is rejected by default.

- The AI never commands the PLC; only the decision engine does, through one PLCService.

- Python writes only M0 / M1 (plus opt-in commissioning bits); never X or Y devices.

- One answer per trigger; never retried; trigger marked answered before the write.

- A late reject is never fired.

- The real PLC never connects automatically; physical test commands need engineer + test mode + arm.

- The software HALT latch and E-stop status input only stop the program answering the PLC. The hardware E-stop must cut power by itself.

- A model never reaches production without the registry gates; training never auto-activates.

- Model proposals are never labels until a person accepts them.

- The ladder is the user's: read and simulate it, never edit it.

## 13. Status: what works, what does not

| Area | In software (fakes) | On the machine |
| --- | --- | --- |
| Per-bottle cycle, FIFO, reject timing, alarms, history, database | works, 25 self-tests | never run |
| PLC link | simulator-tested | read-only link only |
| PLC program | trigger / handshake / reject cycle present | T0/T1 = 15 s / 5 s; no E-stop input, interlock, timeout, watchdog |
| Belt speed, distances, latency | wizard and measurement code work | not measured |
| Two cameras | stations, association, auto-reconnect | one USB 2.0 hub: one 1080p stream |
| Classifier | test macro-F1 0.935 | not validated on EMEET frames; cannot detect missing cap |
| Detector | test mAP50 0.968 (v1) | not validated on EMEET frames; misses full-bottle missing caps (v3 candidate may fix) |
| Operator view / database / export / registry gates / auto-collect | works | not used on a line |
| Segmentation | interface only | no model |

## 14. Troubleshooting

| Symptom | Cause / fix |
| --- | --- |
| "Access is denied" on COM5 | Delta COMMGR holds the port: exit it / delete its driver; one program per port |
| plc_com changes by itself | the running app rewrites it from the Machine dropdown; stop the app before editing settings.json |
| START stays disabled | read the START CHECKS list; typically T0 (15 s in the ladder vs the settings), no camera chosen, PLC not in RUN |
| Second camera gives no image at 1080p | both on one USB 2.0 hub: separate USB 3 ports (alarm CAMERA_BANDWIDTH_PROBLEM) |
| App killed / "paging file is too small" | another job is using RAM / GPU: close it; run tests with CUDA_VISIBLE_DEVICES= |
| Good bottle shown as DEFECT | the model is weak on good bottles (72 examples): collect + review + retrain; check check_margin; see section 7 |
| Bottle shows NOT INSPECTED | it arrived while the PLC was busy: spacing must exceed the travel time (and T0 + T1 after a reject) |
| Screens look different after an update | text size and theme apply at next start |
| A test wrote into my real data | it must not: tests redirect to temp folders; report it |

## 15. Next steps

- Fix the PLC program (presets K15/K5, E-stop input, Y0 interlock, answer timeout) and upload it from the real PLC to compare with the file in the repository.

- Hardware day 1: separate USB 3 ports, measure belt speed and distances, lock camera controls, lighting.

- Collect real data with Auto-collect: many good bottles and missing-cap bottles from several bottles, in the enclosure.

- Retrain and validate on the real camera (gates in section 7); keep the white-background missing-cap images out of the classifier.

- Physical commissioning (section 11) and measure latency (trigger -> decision -> command -> cylinder).

- Later: PLC protocol v2 (several bottles in flight), PC heartbeat watchdog, user login and audit trail, encoder if speed varies.

## 16. Glossary

| Term | Meaning |
| --- | --- |
| PLC / ladder | industrial controller / its program (ISPSoft) |
| Trigger | bottle reaches the photo-eye (M2 on) |
| FIFO | first-in-first-out list of bottles between camera and cylinder, each with a deadline |
| FAULT | "cannot vouch for this bottle"; rejected by default |
| NOT INSPECTED | bottle sensed at X0 but no trigger was possible |
| Recipe | what a complete product looks like (anchor + required parts) |
| Candidate / validated / approved / active | model lifecycle states |
| Hard example | an image where a person corrected the model's pre-label |
| Scene | group of near-duplicate frames of the same bottle, kept whole in a split |
| Evidence | the picture kept for a bottle |
| Encoder | belt-movement sensor; none installed (time is used) |
| ROI | the region of the picture the classifier looks at |

## 17. Document map

| Need | File |
| --- | --- |
| Front page and quick start | README.md |
| This handbook | docs/TEAM_HANDBOOK.md |
| Status + 11 review questions | docs/PROJECT_BRIEF_FOR_REVIEW.md |
| Rules the code relies on | CLAUDE.md |
| Operate and commission | docs/guides/PRODUCTION_HMI_AND_COMMISSIONING.md |
| What the PLC must do | docs/hardware/PLC_LADDER_REQUIREMENTS.md |
| Database / auto-annotation / pages | docs/guides/DATABASE.md, AUTO_ANNOTATION.md, TAB_GUIDE.md |
| Feature truth, scope, plan | docs/roadmap/FEATURE_STATUS.md, CURRENT_SCOPE.md, PROGRESS_PLAN.md, FUTURE_ENHANCEMENTS.md |
| PLC contract and ladder | docs/roadmap/PLC_COMMUNICATION.md |
| Datasets and findings | docs/roadmap/VISION_DATASET.md |
| Hardware and camera plan | docs/hardware/HARDWARE_INTEGRATION.md, CAMERA_PLACEMENT_AND_LINE_PLAN.md |
| Audits | docs/audit/ |

## 18. Using this with an AI assistant

Give it this file (and docs/PROJECT_BRIEF_FOR_REVIEW.md for review questions). Tell it: the system is software-tested only; nothing is validated on the machine; where a value says not measured it must say what to measure; and any change must keep the safety rules in section 12. For code changes also give it CLAUDE.md.