# Gap matrix and implementation record: 2026-10-05

This audit set out to turn the existing application into a coherent, production-oriented inspection platform, without creating a second application. It covers three steps:

- Phase 0: a read-only audit, reconciling CLAUDE.md, CURRENT_SYSTEM.md and the actual code.

- A gap matrix.

- What was implemented in this pass.

Evidence labels (VERIFIED / USER-STATED / INFERRED, and FAKE / SIMULATOR / PHYSICAL) keep their meaning from docs/roadmap/PLC_COMMUNICATION.md.

Everything below marked done is SOFTWARE VERIFIED (self-tests with fake PLC, fake cameras and fake models). Nothing in this pass was run on the physical machine.

## 1. Documentation drift found (Phase 0)

| Claim | Where | Reality in code (VERIFIED by reading and running) |
| --- | --- | --- |
| "No per-bottle logic", "Decision logic PARTIAL: per-frame only" | CURRENT_SYSTEM.md section 3 | decision.py (frame vote, camera fusion, recipe) and machine_cycle.py (one decision per bottle) exist and are FAKE-tested |
| "Sensor trigger, bottle tracking, timing model: NOT IMPLEMENTED" | CURRENT_SYSTEM.md, Machine side | X0 -> M2 trigger, time-based FIFO, deadline REJECT dispatch, timing_problem() exist |
| "PLC layer not integrated with the inspection pipeline or GUI" | CURRENT_SYSTEM.md | Integrated: Machine and Production tabs, one PLCService |
| "Mock PLC, serial transport: NOT IMPLEMENTED" | CURRENT_SYSTEM.md | FakePLC / FakeLadder and SerialTransport exist; first read-only physical link on 2026-10-04 (CLAUDE.md, USER-STATED) |
| "Desktop GUI, 9 tabs", "gui.py ~2.2k lines" | CURRENT_SYSTEM.md | 11 tabs before this pass, 14 after; gui.py ~5k lines |
| "Auto annotation: DEFERRED" | FEATURE_STATUS.md | autoannotate.py + Annotate-tab proposals exist (FAKE-tested) |
| "No held-out test split" | CURRENT_SYSTEM.md | split_train_val_test + test_metrics.json exist for 6 classifier checkpoints |
| missing_cap has 0 positives | both | Still true (VERIFIED 2026-10-05: labels.csv 1,143 rows, missing_cap = 0). Images the user mentioned adding are not in labels.csv |
| train.run() promotes every run | CLAUDE.md | Was true; changed in this pass (see below) |

## 2. Gap matrix (requested capability -> state before -> state now)

| Capability | Before this pass | Now (all SOFTWARE VERIFIED unless noted) |
| --- | --- | --- |
| One machine state | Each widget derived its own (LINE lamp, Production label, Machine label) | machine_state.py: 10 states from one set of facts; Production banner and status-bar LINE lamp show the same value |
| Start readiness checks | PLC connected + timing only | PLC link, PLC RUN, cameras, models, timing (incl. stations and calibration), E-stop, recipe; START disabled until READY |
| Startup / initialising | Line started as soon as models loaded | INITIALIZING until every line camera delivers a first frame (8 s), else CAMERA_DISCONNECTED and not started |
| Orderly STOP | Stop left inspecting / scheduled bottles in the FIFO | Bottles still inspecting or scheduled end as FAULT ("remove by hand"), never silently dropped; run closed in the store |
| Time-based tracking | travel = distance / speed in machine_cycle | tracking.py: PositionSource seam, TimePositionSource (speed plus tolerance gives an explicit uncertainty), EncoderPositionSource refuses to exist (no encoder) |
| Speed calibration wizard | None | Engineer dialog: stopwatch or typed runs, at least 3 runs, mean/min/max/spread, refused above tolerance, timestamped record in settings |
| Sequential (staggered) cameras | All cameras read at the trigger instant | camera_stations (offset mm downstream, wall side, role, per-camera rules); each camera's window is trigger + offset/speed; non-blocking collection; frames bounded by the next bottle's window; CAMERA_ASSOCIATION_FAULT for neighbours too close to separate |
| Camera roles | None | Per-camera decision overrides (judge = parts this camera judges, station_x) in decision.decide(per_camera=...) |
| Layout validation | T0 vs travel only | line_problems(): downstream camera without speed, camera past the reject station, decision ready after the REJECT dispatch time (+ margin) |
| Coded alarms | Free-text list | alarms.py: 25 codes (severity, operator message, action), condition and event alarms, acknowledge (RESET FAULT), dedup, persisted |
| Persistent traceability | Daily CSV | production_store.py: SQLite runs (job, product, recipe hash, model ids, settings), one row per bottle, alarms, evidence images by policy (async writer, disk check); CSV kept |
| Per-stage timings | Total inference ms | capture wait, classification, detection, decision, PLC ms, trigger->decision, trigger->command per bottle; p50/p95 on screen |
| History / reports | None | History page: day, filter, counts, yield, defect distribution, bottles/hour, alarms, evidence viewer, CSV export |
| System health | Partial (Camera tab) | Health page: computer, PLC, cameras, models, storage, alarms -> HEALTHY / WARNING / FAULT |
| Model registry / deployment | Checkpoint folders; training auto-activated | model_registry.py: CANDIDATE -> VALIDATED (held-out test + real-camera note) -> APPROVED -> ACTIVE, archive, deployments.jsonl, rollback; detector activation sha-checked; Models page. train.run() no longer activates by default |
| Critical-class visibility | Hidden in per-defect tables | Registry flags every classifier: missing_cap has no test positives |
| Operator / engineer UX | One dark training GUI, Production 4th in its group | Light industrial theme (dark optional), operator mode (Production / History / Health only), large START / STOP / RESET FAULT, engineer row hidden from operators |
| AI test mode (no PLC) | machine_cycle.py --bench (CLI) | TEST INSPECTION button: same models, decision engine and recipe; result, overlay and timing on screen; files in captures/ |
| Camera-only test | Live tab / Camera tab | CAMERA TEST: chosen line cameras live with brightness / contrast / sharpness / clipping; capture test frame |
| Bandwidth awareness | Hint text | Delivered vs requested resolution shown; CAMERA_BANDWIDTH_PROBLEM alarm |
| Simulator vs real PLC | Small text | SIMULATOR / REAL PLC badge on Production and Machine |
| Self-test pollution | GUI self-test wrote fake bottles into the real project's production CSV | Self-test writes to a temporary folder |

## 3. Not done in this pass (honest list)

| Item | Why / what is needed |
| --- | --- |
| YOLO multi-candidate training, classifier retraining | Must not train before a dataset audit of the new images (and none of the user's new missing-cap images are in labels.csv). Existing tooling (model_bench.py yolo / cls-train / cls-test, vision_data.py) already runs it; candidates then appear in Models |
| Missing-cap hard negatives (bare neck, tamper ring, partial cap) | Needs real EMEET images collected on the machine, annotated (Annotate tab plus proposals), then a detector candidate trained and benchmarked on det-defects |
| Camera auto-reconnect while running | A camera fault makes the bottle FAULT and raises CAMERA_DISCONNECTED; recovery is Stop -> Start (no app restart). An automatic reopen during a run is not implemented |
| Per-camera ROI for the classifier | The classifier ROI is per checkpoint; per-camera ROI for a second view needs a second classifier or ROI-free detection |
| Recipe GUI editor | Recipe is still config.json "inspection" (validated at start) |
| Structured per-subsystem log files | Production events go to SQLite (bottles, alarms, runs) and the CSV; PLC events stay in PLCService.events(). Separate rotating app / camera / AI log files are not added |
| Shift reports, user login | Day-level history only; no users / roles beyond the engineer PIN (a convenience lock, not security) |
| Ladder changes (K150 -> K15, multi-bottle reject FIFO in the PLC) | The ladder belongs to the user; never edited here |

## 4. Physical blockers (unchanged by software)

- T0 / T1 in the saved ladder are K150 / K50 (15 s / 5 s) against the stated K15 / K5; settings.json says 0.75 s. plc_t0_s must equal what is in the PLC. Measure it on the real PLC (Machine tab, T0 value) before any REJECT test.

- One-bottle handshake: M2 stays ON until answered (a REJECT is answered at its dispatch time), and M1 masks triggers through T0 + T1. A downstream camera lengthens the time to answer. Bottles closer than that are NOT INSPECTED (detected and alarmed, not hidden).

- USB: two EMEET Nova 4K on one USB 2.0 hub give one 1080p stream. Use separate USB 3 root ports.

- No measured distances or speed: run the speed calibration and enter the line layout.

- Models: no classifier can detect missing_cap (0 positives); the detector path is the only missing-cap check, and it is unvalidated on EMEET frames.

## 5. Second pass (2026-10-05, afternoon)

| Item from section 3 | Now |
| --- | --- |
| Camera auto-reconnect while running | DONE: background reopen every 3 s, counted, logged; bottles in the gap FAULT (SOFTWARE VERIFIED, fake camera) |
| Recipe GUI editor | DONE: hmi.RecipeDialog, validated against detector classes, refused while running |
| Structured per-subsystem logs | DONE: applog.py, 7 channels, event-based, searchable on Health |
| Shift reports | DONE: History per shift (configurable, night shift across midnight) |
| Active-learning queues (hard negatives, missing components) | DONE: Annotate "Next: missing part" / "Next: doubtful part" |
| PLC ladder requirements | DONE (document + checker): docs/hardware/PLC_LADDER_REQUIREMENTS.md, python -m plc.ladder_check. The ladder itself is the user's; not edited |
| Missing-cap data + retraining | DONE, with a negative result for the classifier (shortcut, REJECTED, images removed, undoable) and a v3 detector candidate that needs machine validation. See README "Missing cap" |
| Per-camera classifier ROI, user login, PC heartbeat | NOT DONE: heartbeat needs a ladder rung first (R13); ROI per camera needs a second classifier; login deferred |

## 6. Third pass (2026-10-05 evening, status 2026-10-06)

| Request | Now |
| --- | --- |
| One direct answer per bottle, no boxes / bars, industry-style | DONE: verdict.py + operator overlay; GOOD / DEFECT: ; boxes and bars only with "Engineer details" |
| Good bottles shown as defective, better confidence | PARTLY: thresholds re-chosen from validation, steady verdict; the model itself is limited by 72 good images, so more real good bottles + retraining are still needed |
| Auto-annotation prediction of defects | DONE / documented: classifier defect pre-labels (Label tab) use the calibrated thresholds; Annotate tab queues; docs/guides/AUTO_ANNOTATION.md (with sources) |
| Simulation check for engineers only, simulator and real PLC | DONE: engineer-only, live PLC snapshot, switch to remove it later (show_simulation_check) |
| Panels full of text hide buttons -> scroll both ways | DONE: every page is a ScrollHost |
| Explain and add the database, exportable | DONE: Database page, CSV + report export, docs/guides/DATABASE.md |
| Restart the system | The app was restarted but stopped by the OS under memory pressure (another training job on the PC) |
| Docs for ChatGPT review | docs/PROJECT_BRIEF_FOR_REVIEW.md |