# System Audit — Bottle Defect Inspection (2026-10-03)

Scope: the whole repository at commit f09fdba (main), including the vision code, models, data, PLC layer, GUI, docs and repo hygiene. Method: every self-check run; safety-critical code read line by line (infer.py, plc/service.py, plc/client.py, MachineTab and TrainTab in gui.py, dataset.py config/split code); every checkpoint's metrics.json, both project configs and labels.csv inspected; roadmap claims checked against the code. Nothing was run on hardware; no PLC or camera was driven.

## 1. Verdict

The software foundation is unusually strong for a project at this stage. The machine is not ready, and four issues block a real inspection cycle.

- Strong: fail-safe PASS/REJECT/FAULT logic, monotonic-clock freshness checks, at-most-once PLC commands, one train/serve crop path, scene-based splits, model provenance with sha256 checksums, and honest documentation that does not overclaim. All 18 software self-checks pass.

- Blocking: (1) the saved ladder turns the reject solenoid on for only about one PLC scan; (2) the ladder lets a bottle pass if Python never answers; (3) the camera verdict is not connected to the PLC; (4) neither model has been validated on images from the production camera.

- Model quality is weaker than the headline numbers. Every Stage 1 checkpoint reports validation macro-F1 = 1.0, but on the held-out test set the active model falsely rejects 10 of 19 good bottles (GOOD recall 0.474).

| Area | Rating | One-line reason |
| --- | --- | --- |
| Inference safety logic (infer.py) | Strong | Every unknown state ends in FAULT; well tested |
| PLC communication (plc/) | Strong (software) | At-most-once, guarded writes, acknowledgement; simulator only |
| PLC ladder (final_year.isp) | Blocking | Reject pulse is about one scan long; no fail-safe default |
| Vision → PLC integration | Not built | The Machine tab sends manual test commands only |
| Stage 1 classifier | Weak | Saturated validation, possible leak, poor GOOD recall on test |
| Stage 2 detector | Fair | Good test mAP50 (0.968) on a small, different-domain set |
| Data | Weak | 72 good images, 0 missing_cap, 2 missing_label scenes |
| GUI | Fair | Works; 2 real bugs; 2,964 lines in one file |
| Docs | Good, drifting | Honest, but 8+ statements are now out of date |
| Repo hygiene | Fair | Whitelist .gitignore works; dead code and ~3 GB of stray local data |

## 2. Self-check results (run 2026-10-03, Python 3.8.0, RTX 3050)

| Check | Result |
| --- | --- |
| dataset.py, train.py --demo, infer.py, inspection_trace.py, detect.py, calibrate.py --demo, charts.py, bench.py, migrate.py --demo, vision_data.py | all pass |
| gui.py --selftest | pass, but it ran against bottle_detection: "9 images, 0 defects" (see H4) |
| python -m plc.test_simulation, -m plc.test_service, -m plc.commissioning --selftest, -m plc.handshake_test --fake | all pass |
| stage2_dataset/review_app.py --selftest | not run; known to fail by design (CLAUDE.md) |
| plc.handshake_test --real, plc.sim_comm_test | not run (needs the ISPSoft simulator) |

The PLC tests fail with ImportError if run as python plc/test_service.py. They must be run with python -m. CLAUDE.md's test list doesn't include them at all.

All of these are software checks with fake captures, fake models, or a fake PLC. None is evidence the hardware works.

## 3. Findings

Severity: Critical = blocks a correct machine cycle or makes it unsafe · High = wrong results or misleading quality claims · Medium = real bug or risk with limited blast radius · Low = hygiene.

### Critical

C1. The reject solenoid fires for about one PLC scan. In the ladder saved at 01:36, net 6 does OUT Y0 and RST M1 on the same rung. M1 enables T0, so on the next scan T0 drops, Y0 turns off, and T1 can never time out. On the simulator, Y0 was ON in 0 of 187 samples on one run and 1 of 126 on another (PLC_COMMUNICATION.md §0, verified from the decoded file). A real solenoid will not actuate on a one-scan pulse. The earlier backups (final_year_2026-10-2-*.~bak) have a correct T0 → T1 → Y0 pulse design. → The ladder owner must restore or fix net 5/6. This is the first thing to fix.

C2. The ladder is not fail-safe: no answer means the bottle passes. X0 sets M2 and then the ladder waits for M0 or M1 indefinitely. Nothing rejects a bottle when:

- Python crashes or hangs, or the link faults;

- the vision result is FAULT (there is no FAULT bit, so FAULT cannot be sent);

- a second bottle arrives within 15 s of a REJECT. M1 is held for the whole T0 (K150 = 15 s), so PLCService._submit correctly refuses with "a command bit is already ON". The second bottle gets no answer and passes.

A QC line must reject by default. → Add a ladder-side inspection timeout that rejects when no answer arrives (or gives a fault output). Map vision FAULT to REJECT (or a dedicated bit). Shorten T0 to the real travel time, or track several bottles in flight (a shift register or FIFO keyed on X0).

C3. Nothing connects the camera verdict to the PLC. MachineTab shows "camera not linked yet: AI result: none" and only sends operator-armed test commands (gui.py:869). There is also no per-bottle decision. At about 15 Hz, one bottle produces many independent frame verdicts, and nothing maps an M2 trigger to "the frames of this bottle". Camera.frames_since(ts) and RECENT_FRAMES are groundwork for this, but no consumer exists. → Build the trigger → inspection window → single decision → send_pass/send_reject loop (Phase 2 of IMPLEMENTATION_PLAN.md).

C4. Neither model has been validated on images from the production camera. The classifier was trained on 2537×1927 black-backdrop JPEGs. The detector was trained on 1780×1000 crops of Iriun-viewer screenshots on a light backdrop. The two datasets share no images (VISION_DATASET.md §4), and the dev cameras default to 640×480 (4:3). The only live detector test had no bottle in view, and it drew false label and bottle boxes on a TV and a door at conf 0.25. → Capture a labelled set of real bottles on the final camera/lighting/backdrop before tuning anything else.

### High

H1. Stage 1 quality claims don't hold up on the held-out test set. All 10 checkpoints report validation macro-F1 = 1.0 (or close). The held-out test for the active model 20260919-164511 (238 images, 23 scenes, HELD_OUT_TEST_RESULTS.md) shows:

|  | Result |
| --- | --- |
| GOOD (all-clear) recall | 0.474: 10 of 19 good bottles falsely rejected |
| skewed_bottle precision | 0.721 (12 FP) |
| tilt_cap precision | 0.688 (10 FP) |
| Exact-match accuracy | 89.1% |

On a line, that means about 1 in 2 good bottles rejected. Contributing factors:

- Possible validation leak. dataset.scene_map groups frames per folder only. 403 cross-folder image pairs are the same frame-run by its own rule (e.g. +ve ↔ Skewed Label, Tilt Cap ↔ Water Level). Every existing checkpoint was trained on that split.

- Hand-tuned live thresholds as low as 0.05 (damaged_bottle, skewed_bottle) and 0.1 for three others. These push toward false rejects and haven't been validated on test or live frames.

- Macro-F1 excludes unscored classes, so "1.0" sits next to missing_cap and missing_label both at 0.0.

→ Retrain on the leak-free split from vision_data.py, choose thresholds on validation only, report test results, and stop quoting validation F1.

H2. The data is too thin for the classes the system claims to detect. 1,143 images ≈ 112 bottles. Good: 72 images in 8 scenes. missing_cap: 0 (disabled). missing_label: 30 images in 2 scenes, so it can't appear in all three splits. 0 multi-label rows: the multi-label design is untested on real data. Viewed samples of skewed_bottle and missing_label look alike (VISION_DATASET.md §2). → Collect far more good bottles (the class that decides false rejects), real missing_cap/missing_label scenes, and genuine multi-defect bottles.

H3. Training while cameras run applies the wrong thresholds (bug). TrainTab.start (gui.py:1623) never stops or reloads the cameras. At the end of train.run() it writes the new checkpoint's thresholds and active_model to config.json (train.py:435-437). The camera thread re-reads config on every scored frame (infer.py:566), but it still holds the old Model. From then on, live verdicts mix old-model scores with new-model thresholds, and nothing warns about it. (AnalysisTab.activate handles this correctly by calling cams.load_model.) Related: every training run promotes itself to the active model even when it scores worse. → Bind thresholds to the loaded model (keep them on Model, or reload the model when active_model changes). Make promotion an explicit step.

H4. The app opens on an empty project, and the GUI integration test runs against it. projects/active.txt = bottle_detection (9 inbox images, no defect columns, no thresholds, no model). Launching run.bat lands on it, and gui.py --selftest reports "9 images, 0 defects". So the "closest thing to an integration test" no longer exercises labelling, defect columns, or analysis against real data. → Switch back to om_bottle, or make the self-test pin its project.

### Medium

M1. The Machine tab event log freezes after 5,000 events (bug). PLCService._events is a deque(maxlen=5000) (plc/service.py:125), and the tab redraws only when len(events) changes (gui.py:886). Once the deque is full its length stays 5000, so the log stops updating while the PLC keeps running. Each bottle produces roughly 10-15 events, so this happens after a few hundred bottles. → Compare the newest event's timestamp, or keep a running counter.

M2. serial_ports(), SerialTransport and SERIAL_FORMATS are defined twice in plc/client.py (lines 113-191 and 192-267, an exact copy-paste). The second copy silently wins. A fix applied to the first copy would be dead code. → Delete one.

M3. Auto-reconnect is on by default in the GUI (plc_auto_reconnect defaults to True, gui.py:553), while the service design says "No automatic reconnect" (plc/service.py:19). Commands are never replayed, and a re-seen trigger is flagged after_reconnect, so this is safe as written. But the stated policy and the actual behaviour differ. → Decide which one is intended and document it.

M4. Capture and inference share a thread. Inference time lowers the capture rate. frame_ts is when read() returned, not when the image was exposed, and DirectShow buffering staleness isn't measured. With a 1.0 s freshness window, a frame can be "fresh" but show a bottle that has already moved on. (Documented in CURRENT_SYSTEM.md.) → Measure glass-to-verdict latency against a real conveyor before setting any timing.

M5. An unexplained hang: a camera reopened after a two-camera session delivered 7 frames, stalled, and the process hung at exit (CURRENT_SYSTEM.md). The FAULT logic reported it correctly, but the cause is unknown. A hang at exit on a line PC means a manual restart.

M6. config.json is written non-atomically (dataset.save_config uses write_text) while the camera thread reads it at ~15 Hz. A torn read raises → "inference failed" → FAULT for one frame. It fails safe, but it can produce spurious FAULTs. (save_labels already uses a temp file and rename; apply the same to config.)

M7. requirements.txt omits pyserial (needed for the "Real PLC (serial)" mode), and ultralytics is commented out (needed for every YOLO mode). Both are imported lazily, so the app starts without them, and those modes then fail with a clear error. → List them, or add a requirements-machine.txt.

### Low

- Dead web app with unauthenticated write endpoints. app.py exposes /api/train, /api/upload/delete, /api/model/activate and others with no auth. Nothing launches it. → Delete app.py + index.html (the web-vs-desktop decision is resolved).

- torch.load(..., weights_only=False) in infer.py:168 runs pickled code from a checkpoint. Acceptable for local files you created; never load a checkpoint from elsewhere.

- gc.disable() app-wide (gui.py:123), with collection on the Tk thread only. It fixes a real freeze, but it's fragile: any new code that blocks the Tk loop also stops GC.

- About 3 GB of local, git-ignored duplicates: All Datasets/ (990 MB, the pre-migration copy), yolo_detection_dataset/ (487 MB), root yolov8n.pt / yolov8s.pt, three stray .log files, and two .git/objects/*/tmp_obj_* garbage files.

- gui.py is 2,964 lines (10 tab classes plus a HMI). MachineTab (about 420 lines) would split out cleanly.

- FINAL_YEAR_BLACKBOOK/06_New_Dataset_Audit/audit_script.py hard-codes E:\MACHINE LEARNING PROJECT\....

- From my push earlier today: I tracked vision_dataset/manifests/* and export_info.json (the images and hash_cache.json stay ignored), but VISION_DATASET.md says vision_dataset/ is "not in git". Either untrack them or update that line.

## 4. Documentation drift

The docs are honest about limits, but these statements are now false:

| Doc | Says | Reality |
| --- | --- | --- |
| CURRENT_SYSTEM.md, FEATURE_STATUS.md | "No held-out test split" | Since the 2026-09-19 checkpoints there is a 238-image test_paths and train.evaluate_test() |
| CURRENT_SYSTEM.md | 9 checkpoints | 10 (20261003-101425, convnext_tiny, trained today) |
| CURRENT_SYSTEM.md | GUI has 9 tabs; PLC layer "not integrated with the … GUI" | 10 tabs, including Machine (PLC) and Annotate |
| FEATURE_STATUS.md | Mock PLC "Not started" | FakePLC / FakeLadder exist in plc/test_simulation.py |
| PLC_COMMUNICATION.md §A, plc/client.py:7 | "SerialTransport: NOT written" | It exists (and twice, see M2); §later in the same doc says so |
| CLAUDE.md | 8 tabs; App at gui.py:53, TABS at :101, self-test at :1836/:1913; "~2.2k lines" | App at :107, TABS at :168, selftest() at :2777, entry at :2961; 2,964 lines; 10 tabs |
| CLAUDE.md | Test list | Missing the four python -m plc... checks |
| VISION_DATASET.md | vision_dataset/ not in git | Manifests now tracked (see Low) |

## 5. What is solid (keep doing this)

- Fail-safe inference. infer.decide turns empty or NaN scores into FAULT. Camera.inspection gives PASS/REJECT only from a fresh score, and a superseded thread can't write into a new session. The self-tests cover every FAULT branch with a real thread and fake captures.

- PLC command discipline. One worker thread owns the link. The "answered" flag is set before the write, nothing is retried, preconditions are re-read immediately before the write, success requires the PLC's acknowledgement, and only M0/M1 are writable (simulator_test_write refuses non-loopback links).

- One crop path for training and inference (dataset.model_input), with lossless PNG cache, ROI scaling via roi_frame, and checkpoint-baked crop parameters for safe rollback.

- Evaluation honesty. Scene splits, disabled zero-positive classes, "unscored" instead of fake 0/1 scores, val_paths/test_paths recorded per checkpoint, and sha256 provenance for the detector.

- Documentation that labels evidence (VERIFIED / USER-STATED / INFERRED / FAKE / SIMULATOR) and never claims hardware success.

## 6. Recommended order of work

- Fix the ladder (C1, C2): restore a real reject pulse; add reject-on-timeout; decide how FAULT maps to a PLC action; size T0 to the measured travel time, or handle several bottles in flight.

- Quick software fixes (H3, H4, M1, M2, M6): each is small and contained.

- Capture production-domain data (C4, H2): the final camera, lighting and backdrop; many good bottles; real missing_cap/missing_label; label it.

- Retrain Stage 1 on the leak-free split (H1) and judge it on test GOOD recall and per-class precision, not validation F1. Re-check the detector on the same images.

- Build the machine loop (C3): M2 trigger → frames after the trigger → one decision per bottle → send_pass / send_reject → record the result in TraceStore.

- Run the simulator handshake (python -m plc.handshake_test --real), then the physical PLC over serial, then the first real cycle with known good and defective bottles, with timing measured.

- Refresh the docs (section 4) and delete app.py/index.html.