# System check, 2026-10-04

Run on the development PC (Windows 11, Python 3.8, RTX 3050 4 GB) immediately before the first hardware integration. Everything below is a software self-check with fakes; none of it is a hardware test.

## 1. Self-checks: 17 / 17 pass (exit code 0)

dataset.py, infer.py, inspection_trace.py, detect.py, decision.py, segment.py, machine_cycle.py, vision_data.py, charts.py, train.py --demo, calibrate.py --demo, stage2_dataset/seg_pipeline.py --selftest, gui.py --selftest (builds all 11 tabs incl. Machine/Production against a fake PLC), plc.test_simulation, plc.test_service, plc.commissioning --selftest, plc.handshake_test --fake. Not run: python -m plc.handshake_test --real / plc.sim_comm_test (need the ISPSoft simulator) and stage2_dataset/review_app.py --selftest (known stale by design, see CLAUDE.md).

## 2. Models: what exists (from models/MODEL_REGISTRY.json)

Stage 1 multi-label classifier, project om_bottle. Validation F1 is saturated (~1.0) -- judge by the held-out test macro-F1.

| Architecture | Epochs | Val macro-F1 | Held-out test macro-F1 | GPU latency (ms) |
| --- | --- | --- | --- | --- |
| EfficientNet-B0 (20260919-164511) | 25 | 1.000 | 0.935 | 32.8 |
| EfficientNet-B1 | 25 | 1.000 | 0.845 | 31.3 |
| ConvNeXt-Tiny | 25 | 1.000 | 0.764 | 19.8 |
| ResNet18 | 8 | 0.933 | 0.773 | 12.6 |
| MobileNetV3-Small | 25 | 0.864 | 0.632 | 21.7 |

Five earlier checkpoints (legacy split, no held-out test) are kept for history. Details: FINAL_YEAR_BLACKBOOK/06_AI_Model_Development/.

Stage 2 detector (bottle/cap/label, 594 images, 1,994 boxes): YOLOv8n (53 epochs, best 33) and YOLOv8s are both trained; provenance in models/stage2_yolo/MODEL_PROVENANCE.json. Observational only; never produces PASS/REJECT.

Not trained: segmentation (label outline). 546 polygons exist but 0 are human-reviewed (seg_annotations.json); seeded polygons are deliberately never exported, so training now would teach the model the seeding heuristic, not the label. Review them in Annotate first (python stage2_dataset/seg_pipeline.py), then train. I did not train it for that reason.

## 3. Findings that matter before the machine is switched on

| # | Finding | Severity |
| --- | --- | --- |
| 1 | Ladder presets T0 K150 / T1 K50 (15 s / 5 s) vs the stated K15 / K5; with them every REJECT masks triggers for ~15 s | High -- fix in ISPSoft (owner's file) |
| 2 | Both EMEETs on one USB 2.0 hub: only one 1080p stream | High -- separate USB ports |
| 3 | Classifier trained on other images/lighting; no data from this enclosure | High -- collect + fine-tune |
| 4 | capture_delay_s is global; staggered cameras need per-camera delay | Medium -- small code change |
| 5 | inspection_to_reject_mm, conveyor_mm_s = 0 (unmeasured) | Medium -- measure |
| 6 | No E-stop input, heartbeat/watchdog, fault latching, alarm history | Medium -- design + ladder |
| 7 | Camera focus/exposure lock written but never tried on the EMEETs | Medium |
| 8 | HMI is an engineer console; no operator-only view | Medium |
| 9 | test split: 238 images / 23 scenes; missing_cap has 0 positives; no real multi-defect images | Documented limitation |
| 10 | Dead code: app.py, index.html (no auth on write endpoints) | Low -- delete on your OK |