# Bench test: PLC -> vision -> decision engine -> PLC

How to check that the vision system decides PASS / REJECT on its own and the PLC acts on it, at home or on the bench, without running the full line. Written 2026-10-04 while connecting the real DVP-SS2.

## 1. What is connected to the decision engine

Only the Production tab drives the decision engine. The other tabs only look or test.

| Tab | Talks to PLC? | Uses the decision engine (decision.py)? | Purpose |
| --- | --- | --- | --- |
| Production | Yes: reads M2 (+X0), writes M0/M1 | Yes (machine_cycle.Inspector -> decision.decide()) | The real automatic line |
| Machine | Yes: reads everything; operator-armed PASS/REJECT only | No (the operator is the "decision") | Commissioning, I/O view, manual handshake test |
| Live | No | No (classifier/YOLO verdict on screen only) | Watching what the models see |
| Camera | No | No | Measuring camera settings (sharpness, FPS) |

The automatic path:

bottle in front of the photo-eye (X0)
  -> ladder net 3: X0 rising edge SETs M2
  -> PLCService sees M2 rise -> Trigger #n
  -> Inspector: frames captured strictly AFTER the trigger, every line camera
  -> classifier (+ YOLO) -> decision.decide(): frame vote, camera fusion FAULT > REJECT > PASS
  -> PASS  : M0 now                          -> ladder: C0 +1, M2 cleared
  -> REJECT: M1 at trigger + travel - T0     -> ladder: T0 -> Y0 (solenoid) for T1, C1 +1
  -> FAULT : rejected by default (fault_action = "REJECT")
  -> daily production CSV

## 2. Simulator switches vs the real PLC

The Machine tab's SIMULATOR TEST switches (X0 X1 X2 M2 M0 M1, Pulse X0) are enabled only for the ISPSoft simulator (TCP 127.0.0.1). On the real PLC they stay grey. This is intentional:

- X inputs cannot be forced over Modbus on a real PLC. Every scan the PLC reloads X0..X17 from the input terminals, so a written X0 would be overwritten within one scan. A software X0 switch would not work.

- Writing M0/M1/M2 directly on the real machine bypasses the handshake rules (one answer per trigger, preconditions re-read, never retried) and can fire the reject cylinder. The code allows only M0/M1, and only through the guarded PASS/REJECT path.

- For the real PLC there is a separate OPERATOR TEST row (added 2026-10-04 on request, for commissioning and testing; remove it later by setting "plc_operator_controls": false or deleting the row):

| Button | Writes | Needs in the ladder |
| --- | --- | --- |
| Conveyor START | M10 pulse (1, 0.3 s, 0) | an M10 contact in parallel with X1 in net 1 (SET Y1) |
| Conveyor STOP | M11 pulse | an M11 contact in parallel with X2 in net 2 (RST Y1) |
| Virtual bottle | M2 <- 1 | nothing (the ladder already clears M2 on M0/M1) |

- Enabled only when settings.json "plc_operator_controls": true, the link is CONNECTED, the PLC is in RUN and the row's arm box is ticked; arm clears after every press (one press = one write). Virtual bottle is refused while M2, M0 or M1 is ON. Y0/Y1 and X are still never written. Software STOP is not an emergency stop. Code: PLCService.operator_write (plc/service.py), address_map.OPERATOR_BITS; test: test_operator_write_is_opt_in_and_narrow in plc/test_service.py.

## 3. How to bench test at home with the real PLC (no code change)

Use the real input, not a software switch: the trigger is then exactly the one the line will see.

- Free the COM port: in COMMGR, Delete Driver, then exit COMMGR (check the tray); go offline in ISPSoft. One program per COM port. The PLC keeps running its program without ISPSoft.

- Link: Machine tab -> Real PLC (serial) -> COM5 (Silicon Labs CP210x), 9600, 7E1, station 1 -> Connect. Header must show PLC ON; X0/M2/Y0 show live ON/OFF; PLC must be in RUN.

- Trigger source (pick one):

  - the photo-eye itself: pass a bottle or your hand through the beam; or

  - a push button / toggle wired in parallel to X0 (same 24 V common as the sensor) = a bench "bottle" switch. Check in the Machine tab: X0 ON -> M2 ON.

- Manual handshake first (Machine tab): trigger -> tick arm -> PASS (M0). Expect ACKED, M2 off, C0 +1. Repeat with REJECT (M1): M1 stays ON through T0, then Y0 pulses for T1, C1 +1. With the cylinder connected and air on, REJECT moves the cylinder. Turn the air off for a dry test.

- Automatic (decision engine): Production tab -> pick the camera(s) -> Start line. Put a good bottle in view and trigger: expect PASS -> M0. Put a defective bottle (or cover the camera) and trigger: expect REJECT/FAULT -> M1 -> Y0. Every bottle appears once in the Production table and the daily CSV.

- Safety checks: Production STOP button latches (no M0/M1 sent while halted); 3 FAULT bottles in a row latch (fault_latch_after). Hardware E-stop input is not configured yet (estop_device unset).

## 4. Settings that must be right before step 5

| Setting (settings.json) | Now | Needed |
| --- | --- | --- |
| plc_mode / plc_com | serial / COM5 | as is (matches COMMGR: RS-232 ASCII 9600 7E1 station 1) |
| plc_t0_s, plc_t1_s | 0.75 / 0.25 | must equal the T0/T1 presets in the PLC. Saved ladder has T0 K150 = 15 s, T1 K50 = 5 s |
| inspection_to_reject_mm, conveyor_mm_s | 0 / 0 (not measured) | measure; conveyor ~1524 mm in 14-16 s = ~95-110 mm/s |
| line_cameras | [2, 3] | indices of the EMEET cameras actually enumerated |
| camera_controls | not set (autofocus/auto-exposure ON) | lock focus/exposure once mounted, to stop flicker |
| estop_device | not set | a spare X input wired to the E-stop NC contact |

## 5. Evidence labels

Everything above the PLC link was run as software self-tests with fakes (python -m line.machine_cycle, python -m plc.test_service). The real-PLC link on COM5 was proven by COMMGR/ISPSoft (2026-10-04), not yet by this software: record the first successful Machine-tab connection and handshake as PHYSICAL evidence in docs/roadmap/PLC_COMMUNICATION.md.