# Hardware Integration

Status 2026-10-06. Parts of this page are older than the code. Current truth: the ladder can be read (it is not encrypted; python -m plc.ladder_check, python -m plc.ladder_sim), the PLC address map is in plc/address_map.py and verified against the simulator and a read-only real-PLC link (2026-10-04), timing / PLC / camera code exists. What is still not measured on the machine: belt speed, distances, T0/T1 in the real PLC, capture and actuator latency, camera placement and lighting. See PLC_LADDER_REQUIREMENTS and PRODUCTION_HMI_AND_COMMISSIONING.

The physical system the software will eventually be connected to. Nothing on this page has been verified against the real machine from within this repository. The hardware list below is what the project owner describes; the repository itself contains no measurements, drawings or test logs for it.

## Hardware context (as stated by the project owner)

| Item | Role |
| --- | --- |
| Conveyor | Moves 250 ml Bisleri bottles past the inspection point to the reject location |
| 2 x EMEET NOVA 4K cameras | Image acquisition. Both enumerate and stream on the dev PC (see Camera notes: one 1080p stream per USB 2.0 hub); not yet mounted on the machine |
| Photoelectric bottle sensor | Detects a bottle arriving (not represented in software) |
| Delta DVP-series PLC, programmed in ISPSoft | Deterministic machine control |
| Festo DSNU cylinder + 5/2 solenoid valve | Reject actuator |
| Controlled LED lighting | Consistent illumination (not represented in software) |
| Inspection enclosure | Controlled imaging environment |

## What exists in the repository for this hardware

| Item | State |
| --- | --- |
| plc file/delta_sim_test.py | Standalone manual sender (Modbus ASCII over TCP to 127.0.0.1:10002, station 1) for the ISPSoft simulator. Not part of the application |
| plc file/final_year/ | The user's ISPSoft project. The .isp is a header + raw-deflate text (decoded by plc/ladder_check.py): 7 networks, T0 K150 / T1 K50. The .ini now names COM5 (the real PLC) |
| Camera code (infer.py) | Index probing with device names, requested capture mode, camera lock, rotate, auto-reconnect while running |
| Timing / trigger / PLC code | machine_cycle.py, tracking.py, plc/ (trigger, deadline FIFO, M0/M1, Y0 observation); software-tested only |

### PLC addresses

Superseded: the devices (X0 photo-eye, X1/X2 start/stop, M0 PASS, M1 REJECT, M2 trigger, Y0 reject, Y1 conveyor, T0/T1, C0/C1) come from the ladder's own device comments and are listed in plc/address_map.py with their verification level. What the real PLC actually holds is still unconfirmed: upload it from the PLC in ISPSoft and compare it with the file in the repository.

## Software/PLC boundary (conceptual -- design intent, not implemented)

A result handshake will be needed so that a late, missing or unacknowledged result leads to a safe outcome chosen in the PLC. The elements to design and verify once the PLC is accessible: a result signal from software, an acknowledgement, a heartbeat/watchdog in each direction, a result timeout, a fault/reset path, and a counter strategy. The result signal (M0 / M1) and acknowledge (M2 / M0 cleared) exist; a heartbeat / watchdog and a PLC-side timeout do not (needs a ladder change, PLC_LADDER_REQUIREMENTS R12 / R13). See INDUSTRIAL_ARCHITECTURE.md for the responsibility split (software decides what; the PLC decides how the machine responds).

## Timing: the equation that decides whether this works

A bottle that is rejected must still be at the reject actuator when the actuator fires.

available travel time  =  (distance from sensor/camera to reject position)  /  (conveyor speed)

The whole software + machine chain must complete inside that time, with margin:

capture
+ preprocessing
+ inference
+ decision
+ communication (PC -> PLC)
+ PLC scan / response
+ actuator response (valve switching + cylinder travel)
        <  available travel time

### What is known and what is not

| Quantity | Value in the repository |
| --- | --- |
| Sensor/camera-to-reject distance | Unknown -- not recorded |
| Conveyor speed | Unknown -- not recorded |
| Capture latency | Unmeasured. Frame timestamps are taken when read() returns, not at exposure |
| Preprocessing + inference | Measured only on the development laptop: YOLOv8n ~19 ms model-only / ~29 ms end to end on an RTX 3050 Laptop GPU (batch 1, under memory pressure). Not measured on the target machine or with real frames |
| Decision | Implemented; trigger -> decision time is now measured per bottle (timings in the record, p50 / p95 on the Production page), but only with fakes |
| PC -> PLC communication | Unmeasured |
| PLC scan / response | Unmeasured |
| Valve + cylinder response | Unmeasured (a datasheet value is not a measurement of this installation) |

Actual hardware measurements are required before any claim that the chain fits. The timing model is Phase 5 of PROGRESS_PLAN.md, and nothing in the software should hard-code a value for these quantities until they are measured.

## Camera notes

- The Live tab captures at driver defaults. The Production line requests line_capture_wh (default 1920x1080) with line_fourcc (MJPG) and shows what the camera actually delivers.

- Exposure, focus, white balance and rotation can be locked (camera_controls, Camera page); a dead line camera is reopened automatically while the line runs; there is no hardware trigger.

- On Windows time.monotonic() has ~15.6 ms resolution; frame sequence numbers give the strict ordering.

- EMEET Nova 4K, measured on the development PC 2026-10-03 (cameras on the desk, NOT mounted on the machine): DirectShow order is 0 = USB2.0 HD UVC WebCam (laptop), 1 = Iriun Webcam (virtual), 2 and 3 = EMEET SmartCam Nova 4K (infer.camera_names() reads this; the Production tab picks the EMEETs by default). One EMEET alone: 640x480 default, 1920x1080 MJPG 30 fps, 3840x2160 MJPG ~14 fps, ~1 s to open. Both at once: only one stream at 1920x1080 or 1280x720 (the second returns no frames, either open order, DSHOW or MSMF). Both stream at 640x480 (~30 fps each). Cause: both are on the same Generic USB 2.0 hub (VID_1A40&PID_0101, Hub #3, ports 2 and 3); UVC isochronous bandwidth is reserved per stream. Fix: plug each EMEET into its own PC USB port (different root hub, ideally USB 3). Until then set line_capture_wh to [640, 480] for a two-camera line.

- Placement, height, angle, field of view, lighting, reflection, focus and exposure in the current enclosure: not started -- needs the cameras mounted on the machine. Start with one camera (1920x1080), check bottle/cap/label visibility in the Production tab's live view and the YOLO boxes on the evidence image, then add the second.

## First hardware checklist (for Phase 5-8)

- Measure distances and conveyor speed.

- Export the ISPSoft ladder + I/O list; verify PLC model and protocol.

- Measure sensor-to-capture and capture latency with the real camera.

- Measure PC -> PLC round trip and actuator response.

- Compute the budget; only then build reject timing.

- Validate with known good and deliberately defective bottles, including failure injection.