# Industrial Architecture

Status 2026-10-06: most layers now exist in software (tags below). Physical validation is still missing for all of them. See CURRENT_SYSTEM.md for what exists and FUTURE_ENHANCEMENTS.md for what is deferred.

## Layers

JOB / RECIPE                 which product, which rules; one versioned configuration     [PARTIAL: job + product per run, recipe editor + hash; no wizard]
      |
PRODUCT CONFIGURATION        classes, dimensions, tolerances, thresholds                  [PARTIAL: per-project config.json]
      |
CAMERA + LIGHTING            devices, exposure/gain/focus, controlled illumination        [PARTIAL: camera lock, rotate, reconnect; lighting not in software]
      |
ACQUISITION / TRIGGER        sensor-triggered capture, frame sequence + timestamps        [IMPLEMENTED in software: X0 -> M2 -> frames after trigger, per-camera window]
      |
PREPROCESSING                ROI crop, resize, normalisation                              [IMPLEMENTED: dataset.model_input]
      |
AI + TRADITIONAL VISION      detection / classification / segmentation; measurement       [PARTIAL: classifier + YOLO in the line; segmentation untrained; no traditional vision]
      |
INSPECTION / DECISION ENGINE per-bottle aggregation, rules, multi-camera fusion,
                             PASS / REJECT / FAULT with reasons                           [IMPLEMENTED in software: decision.py + verdict.py for the screens]
      |
TRACEABILITY                 inspection record, evidence, counters, history               [IMPLEMENTED in software: SQLite, evidence, alarms, logs, export]
      |
PLC INTERFACE                result delivery, handshake, watchdog, timeout                [IMPLEMENTED: M0/M1/M2 handshake; NO watchdog / timeout]
      |
MACHINE CONTROL              deterministic logic: conveyor, interlocks, reject timing     [PLC: ladder read + simulated; presets wrong, no E-stop / interlock / timeout]
      |
REJECT / ACTUATOR            cylinder + valve                                             [HARDWARE]

## Responsibility boundary

AI decides what the object / defect is. The PLC decides how the physical machine responds.

| Software (this repository) | PLC |
| --- | --- |
| Perception: detect/classify what is in the frame | Deterministic machine control |
| Inspection and the decision (PASS / REJECT / FAULT) with reasons | Conveyor start/stop and speed |
| Evidence and traceability | Sensor inputs and their debouncing |
| Detecting its own failure (FAULT) | Actuator outputs, reject pulse timing |
| Delivering the result to the PLC | Safety interlocks and emergency behaviour |
|  | Acting safely if the software stops responding (watchdog / timeout) |

Consequences of this split:

- The PLC must never depend on the PC for safety. A missing, late or FAULT result must lead to a defined safe outcome decided in the PLC (for example, treat as reject). Which outcome is correct is a machine-design decision not yet made.

- The software must never claim PASS when it does not know. That is the purpose of the FAULT state and freshness checks already in infer.py.

- Timing across the boundary must be measured, not assumed -- see HARDWARE_INTEGRATION.md.

## Data flow for one bottle (target)

sensor edge -> trigger -> capture N frames (each: camera_id, session, seq, monotonic ts)
   -> detect/classify each frame -> aggregate per bottle (and across cameras)
   -> decision engine -> PASS | REJECT | FAULT (+ reasons, hits, model id)
   -> InspectionRecord (id, timestamps, evidence) -> PLC result -> reject at the right moment

The Frame / Inspection / InspectionRecord contracts in infer.py and inspection_trace.py are the first pieces of this flow that exist.

## Design rules

- Fail safe: unknown is FAULT, never PASS.

- Time is monotonic and explicit; wall-clock time is for humans and reports only.

- Every result carries enough metadata (camera, session, sequence, model) to trace it back.

- The PLC owns real-time behaviour; software never tries to be a hard-real-time controller.

- Add platform features (recipes, registry, dashboards) only after the minimum machine is proven.