# ABVIS Industrial Upgrade Plan

Audit date: 2026-10-13 · Scope: whole codebase, configuration, models, datasets, production record, logs. Target: a commercial-grade inspection station that decides on each bottle with high confidence, says where the defect is, and never passes a bottle it could not inspect.

## Implementation status (updated 2026-10-13, after the first build round)

All items below are software, verified with self-tests (fakes). None is a hardware test.

| Plan item | Status | Where / evidence |
| --- | --- | --- |
| 0.1 FAULT -> PASS blocks Start | Done | machine_cycle.safety_problem in line_problems; machine-cycle self-test |
| 0.2 Model references | Done (restored) | stage2_best.pt + MODEL_PROVENANCE.json + classifier 20260919-164511 copied back from models_archive/previous/; detector self-test passes |
| 0.3 Live threshold shift | Open | still in settings.json (threshold_shift -0.3); now every settings change is audited (see below) |
| 0.4 run.bat | Done | from vision import dataset; migrate only for an unmigrated checkout; Python 3.8 text |
| 0.5 Duplicate PLC classes | Done | second SerialTransport / serial_ports copy removed from plc/client.py |
| 0.6 White-background missing-cap images | Open (data decision) | still in om_bottle; the 2026-10-08 B0 candidate has the shortcut (missing_cap ~1.00 on every real frame) |
| 0.7 Archive tool | Done (not run) | python -m tools.archive_models (plan by default, never moves active models, restore) |
| 1.1 One high-resolution clock | Done | line/clock.py; 702 / 1000 distinct back-to-back stamps (was ~1) |
| 1.2 Bracketed trigger + uncertainty | Done | Trigger.edge_lo / est_mono / uncertainty_s; reject scheduled from est_mono; Bottle.trigger_unc_ms |
| 1.3 Fast trigger poll | Done | fast poll reads Y0 while the line runs; status read 1 Hz; test: Y0 pulse seen with 0 status reads |
| 1.4 AI off the deadline thread | Done | Inspector.async_ai; test: 0.25 s model, every poll() < 50 ms |
| 1.5 LED latency test | Open (needs hardware) |  |
| 1.6 Three-zone decision + min bottle frames + unsure KPI | Done | decision rules pass_max, min_bottle_frames_frac; PRODUCTION_RULES; UNSURE counter tile + alarm |
| 1.7 Calibration, threshold rule, best epoch | Done | train._best_threshold, fit_temperature, val-loss tie-break, robustness_aug; 2-epoch real run verified |
| 1.8 Production-camera data | Open (needs the line) |  |
| 2.1 PLC bottle FIFO | Open (ladder change) |  |
| 2.2 Defect classes / localisation | Engine done, data bootstrapped | recipe defects + any_of; Decision.locations -> DB/CSV/evidence; vision.defect_bootstrap --write proposed 389 defect boxes (229 label_damaged, 71 label_skewed, 89 cap_tilted) for review; 620 images listed for hand-drawn regions |
| 2.3 YOLOv8s v4 training | Open (after review) |  |
| 2.4 Detector-guided crop | Open |  |
| Extra: annotation export bug | Fixed | unannotated images were exported as empty (background) labels; annotate.exportable |
| Extra: settings audit trail | Done | every saved settings change logged old -> new; safety keys at WARNING |

Self-check: python selfcheck.py 22 / 22; full set passes except the GUI self-test, which was not run while the application was open (it rewrites settings.json).

## 0. Executive summary

| Area | Verdict | Evidence |
| --- | --- | --- |
| Software control logic (FIFO, handshake, deadlines, alarms, record) | Sound | python selfcheck.py: 18/18 pass (fakes only) |
| Configuration | Unsafe now | fault_action: "PASS", threshold_shift: -0.3, detector gates lowered to 0.2 |
| Model files | Broken now | active_model 20260919-164511 and stage2_best.pt are in models_archive/ |
| Real-line workflow | Does not work as designed | production.db: 200 bottles → 0 PASS, 4 REJECT, 196 FAULT; 167 never inspected |
| Camera timestamps | Consistent, but not exact | 15.6 ms clock ticks; stamp excludes driver latency; trigger stamped after the serial reply |
| Classifier | Good on its own test set, fails off-domain | test macro-F1 0.949 (B0); scores sit at 0.1–0.7 on the EMEET camera |
| Detector | Good on its own test set, fails off-domain | test mAP50 0.979 (v8s v3); near-full-frame boxes on the EMEET camera |
| Training data | Not enough for commercial use | ~106 scenes; good = 8 scenes; missing_label = 2 scenes; another camera; no multi-defect images |
| Defect localisation | Not implemented | the detector knows bottle / cap / label only |

The main conclusion: more epochs will not make this system industrial. The classifier already reaches a perfect validation score at epoch ~12. What limits it is data from the production camera, the crop, the threshold rule and the PLC handshake. Of the current models, only the YOLOv8s detector would gain from more epochs (best epoch 70 of 80, validation loss still falling).

## 1. Method

- Read every package (vision/, line/, registry/, ui/, plc/, tools/), CLAUDE.md, the old README, settings.json, projects/om_bottle/config.json, run.bat and requirements.txt.

- Loaded every classifier checkpoint (model.pt header) and read metrics.json / test_metrics.json.

- Read the YOLO results.csv, args.yaml, candidate_*.json and the dataset YAMLs.

- Counted labels.csv and unified_manifest.csv per class and per scene.

- Grouped projects/om_bottle/production/production.db rows by result and reason.

- Read captures/*/result.json (bench captures on the EMEET cameras).

- Measured this PC's clocks: time.get_clock_info('monotonic') → GetTickCount64(), resolution 0.015625 s; perf_counter → QueryPerformanceCounter(), resolution 1e-07 s.

- Ran python selfcheck.py (CPU): 18/18 pass.

## 2. Workflow verification: designed vs actual

### 2.1 Step by step

| Step | Designed | Code | Real-line result |
| --- | --- | --- | --- |
| Bottle sensed | X0 → ladder sets M2 | plc/service.py::_observe | Works. X0 rises without M2 are detected and recorded (BOTTLE_UNTRIGGERED) |
| Trigger accepted | One bottle per M2 rise | ladder net: M2 cannot rise while M2 or M1 is ON | 156 bottles: "M2 already ON: previous bottle not answered"; 11: "M1 ON: reject cycle holds M2 reset" |
| Frames collected | Frames after the trigger, inside the camera window | Inspector.begin/poll | Works in code. 9 bottles "no bottle found" (detector off-domain) |
| AI decision | Vote + fusion | decision.decide | 4 REJECTs, each listing several defects at once (off-domain scores) |
| PLC answer | PASS → M0, REJECT → M1 at travel − T0 | _decided/_send/_due | Throughput collapses (2.2) |
| Fault handling | FAULT → REJECT | fault_action | Configured as PASS |
| Halt latch | 3 consecutive FAULTs halt | fault_latch_after | Worked: 7 bottles "line halted" |
| Record | One row per bottle | production_store | Works: 200 rows, reasons recorded |

### 2.2 Why 78 % of bottles were not inspected (throughput)

Settings in force: travel = 1028 mm / 108.2 mm/s = 9.5 s, T0 = 5.0 s, T1 = 2.5 s, window = 3.9 s, 13 frames.

PASS bottle:   M2 ON from trigger until answered ≈ 3.9 s window + AI ≈ 4.3 s
REJECT bottle: M2 ON until M1 is sent at 9.5 − 5.0 = 4.5 s, then M1 held T0 + T1 = 7.5 s → ≈ 12 s

The ladder handles one bottle at a time (one M2, one T0). So the line can take at most about 14 bottles/min if every bottle is good and about 5/min with rejects. A 250 ml line runs at 60–600 bottles/min. At 108 mm/s with a 100 mm pitch a bottle arrives every ~0.9 s. A 3.9 s window therefore spans about four bottles, so the camera-association check must fault.

The fix is architectural, not a tuning change (Phase 2):

- The PLC keeps a FIFO, not one M2 bit. The PC answers every bottle at once with its id and verdict (e.g. a D-register ring buffer or M-bit shift register). The PLC tracks each bottle to the cylinder by an encoder count (preferred) or its own per-slot timers, and fires Y0 by itself. The PC no longer waits until travel − T0.

- Watch window ≤ bottle pitch / belt speed (e.g. ≤ 0.3–0.5 s), with 3–5 frames, not 13 frames in 3.9 s.

- The decision deadline is "before the bottle reaches the next station", not "before travel − T0".

### 2.3 Other workflow defects

| # | Defect | Where | Fix |
| --- | --- | --- | --- |
| W1 | fault_action: "PASS" | settings.json | Set "REJECT", and make START refuse PASS unless monitor_only is on |
| W2 | Active classifier and default detector were archived | config.json active_model, models/stage2_yolo/stage2_best.pt | Restore them or activate new candidates through the Models page. START must show "model file missing" |
| W3 | Model inference runs on the machine-cycle deadline thread | Inspector.poll → _score inside MachineCycle._run | Run AI on a worker pool. _due() must never wait behind a 150–300 ms model call (reject_tolerance_s is 0.3 s) |
| W4 | run.bat re-runs calibration on every launch | python -c "import dataset as D..." (module moved) | Use from vision import dataset as D |
| W5 | run.bat runs migrate --run whenever All Datasets/ exists | All Datasets/ was re-extracted | Remove the step or check projects/om_bottle first |
| W6 | run.bat says Python 3.10+; code targets 3.8 | run.bat | Pick one version (3.11 recommended, §9) |
| W7 | Duplicate serial_ports / SerialTransport definitions | plc/client.py lines 113 and 192 | Delete the second copy |
| W8 | Every yolo_export*/data.yaml points to a missing folder | E:/MACHINE LEARNING PROJECT/Datasets/... | Re-export, or write relative paths |
| W9 | plc_com is COM10 (Bluetooth); the PLC was on COM5 | settings.json | Pin the port by USB-serial adapter serial number, not by COM index |
| W10 | Archived models cannot be rolled back | the registry scans only projects/<slug>/models | Teach the registry about the archive (§8) |

## 3. Camera layer and timestamp audit

### 3.1 How it works today

grab thread:   t_read = perf_counter(); got, frame = cap.read(); t_grab = monotonic()   ← stamp
               rotate → seq += 1 → recent.append(Frame(cam, t_grab, seq, frame))       (deque of 12)
PLC thread:    read_many([X0, M2, M0, M1]) over 9600 baud → now = monotonic() → Trigger(seen_mono=now)
Inspector:     window start = seen_mono + offset/speed + delay;  use frames with start < ts < next start

What is correct: one clock (monotonic) on both sides; strict > against the coarse tick; the session and seq counters make frames unique; the next bottle's window bounds this bottle's frames; stale frames FAULT; restart-safe threads.

### 3.2 Findings

| # | Finding | Effect | Severity |
| --- | --- | --- | --- |
| T1 | time.monotonic() on Python 3.8 / Windows is GetTickCount64, 15.6 ms steps | Neighbouring frames share a stamp; a frame up to 15.6 ms before the trigger can look "after" it, or the reverse | Medium |
| T2 | The frame stamp is taken when read() returns, not at exposure | Each frame is stamped late by exposure + USB transfer + MJPG decode + DirectShow buffering (typically 40–150 ms on a UVC webcam; never measured here) | High |
| T3 | The trigger is stamped after the serial reply arrives | It includes the ladder scan, up to one poll period and the transaction time (~30–40 ms per Modbus ASCII transaction at 9600 baud) | High |
| T4 | status_period_s = 0.15 but a full read_status() is ~6+ transactions (~200–300 ms at 9600) | Almost every poll becomes a full status read, so trigger detection lags by up to ~300 ms with jitter. At 108 mm/s that is ~32 mm of bottle position | High |
| T5 | RECENT_FRAMES = 12 (~0.4 s at 30 fps) | If the deadline thread is blocked by model calls for more than 0.4 s, frames of the window are lost | Medium |
| T6 | EMEET Nova 4K: rolling shutter, autofocus, auto-exposure (no camera_controls locked except rotate) | A moving bottle is sheared by the rolling shutter, which looks like skewed_bottle. Focus and exposure drift between bottles | High |
| T7 | Two cameras share one USB 2.0 hub (measured: only one 1080p stream) | Frame drops and timestamp gaps on the second camera | High |
| T8 | No end-to-end timing test exists | Nobody knows the true camera latency or trigger latency | High |

Answer to "does the camera layer work with exact timestamps?" It is internally consistent and safe: it never uses a frame from before the trigger by more than one clock tick, and it never treats a stale frame as current. It is not exact. Frame stamps run late by an unmeasured driver latency, trigger stamps run late by an unmeasured and variable serial latency, and both are quantised to 15.6 ms. At the current belt speed the combined error can reach ~30–50 mm of bottle travel.

### 3.3 Fixes

Software (Phase 1):

- One high-resolution clock. Add line/clock.py with now = time.perf_counter and use it for every frame stamp, trigger, deadline and PLC event (replace time.monotonic() in vision/infer.py, plc/service.py, line/machine_cycle.py, line/tracking.py). Keep the strict > rule.

- Bracket every stamp. Frame: record t_before_read and t_after_read. Trigger: record the request-send time and the reply time of the poll that saw the edge, and the reply time of the previous poll. The edge happened between prev_poll_send and this_poll_send. Store trigger_ts = midpoint and trigger_uncertainty = half the interval, and feed that uncertainty into tracking.association_problem.

- Subtract measured latency. settings.json camera_latency_ms per camera and trigger_latency_ms, measured by the test in step 6. frame.ts_exposure = t_after_read - camera_latency.

- Split the fast trigger poll from the slow status poll. Trigger loop: read only X0 + M2 (one transaction) as fast as the link allows. Status read: at most 1 Hz, and never while a bottle is pending.

- RECENT_FRAMES derived from the window: ceil((window_s + 0.5) × fps). Move model calls off the deadline thread (W3).

- Timing commissioning test (tools/timing_test.py, new). An LED driven by a PLC output, placed in the camera view and also wired into an X input: the PLC toggles it, the PC records the X edge time (trigger path) and the first frame where the LED is lit (camera path). Repeat 200 times and report the mean and p99 of each latency. START refuses when no measurement younger than 30 days exists.

Hardware (Phase 3, required for commercial use):

- Hardware-triggered, global-shutter industrial cameras (USB3 Vision / GigE Vision with an opto-isolated trigger input, e.g. Basler ace 2, Hikrobot MV-CS, FLIR Blackfly S). The photo-eye fires the camera directly, so the frame is the bottle and the timestamp is the camera's hardware timestamp. This removes T1–T4 and T6.

- Fixed lens, locked exposure, strobed or constant LED lighting (back-light for fill level and clear-bottle defects, dome or diffuse front-light for labels and caps), inside an enclosure.

- One camera per USB3 controller or a PoE switch for GigE.

- An encoder on the conveyor (tracking.EncoderPositionSource is already the seam), so position is counted, not estimated from time.

- Faster PLC link: Modbus RTU at 115200 or an Ethernet module (Modbus TCP), or move tracking into the PLC (§2.2).

## 4. Model audit

### 4.1 Classifier (Stage 1)

| Check | Result |
| --- | --- |
| Held-out test (best: B0 20261008-081732) | macro-F1 0.949, 0 false passes, 9 false rejects of 226 |
| Weakest class | tilt_cap: P 0.67, R 0.82 on every architecture (it needs geometry, not a global score) |
| Validation | saturated (macro-F1 1.000 from epoch 12 on every run) → cannot select an epoch or a threshold |
| Threshold rule | train.evaluate returns the first F1-optimal threshold of 0.05…0.95, i.e. the lowest. Hence 0.05 on most classes |
| Best-epoch rule | first epoch to reach the best macro-F1 (strict >), i.e. the earliest perfect epoch, with the smallest margins |
| Calibration | none. A 0.9 is not "90 % sure" |
| Crop | fixed ROI [826, 0, 841, 1927] on a 2537×1927 landscape frame, rescaled onto a 1080×1920 portrait frame at runtime. The strip's aspect drops from 0.44 to 0.19, so the model sees a differently scaled bottle |
| Augmentation | ±5° rotation, ±3 % scale, brightness/contrast. No blur, noise, colour shift, perspective, rolling-shutter shear, background change or occlusion |
| Shortcut risk | the 22 white-background missing-cap images are back in om_bottle (the 2026-10-05 candidate learned "white background = missing cap") |
| On the production camera | undecided scores (0.1–0.7 on every class), several defects at once |

More epochs? No. With a saturated validation set, extra epochs only memorise the training scenes harder. Train longer only after the data and the validation set change (Phase 1–2). Then use 60–100 epochs with early stopping on validation loss and a validation set from the production camera.

### 4.2 Detector (Stage 2)

| Check | bench_yolov8s_v3 | bench_yolov8n_v3 |
| --- | --- | --- |
| Epochs run / best | 80 / 70, val box loss still falling at epoch 80 | 48 / 28 (early stop, patience 20) |
| Val mAP50 / 50-95 | 0.984 / 0.859 | 0.982 / 0.853 |
| Test mAP50 / 50-95 | 0.979 / 0.818 | — |
| Weakest class | label (test mAP50-95 0.62) |  |
| Latency (RTX 3050) | ~10.9 ms | lower |
| On the production camera (old stage2_best.pt) | bottle and cap boxes covering nearly the whole frame at conf 0.6–0.7 |  |
| Missing-cap evidence | v3 test set has no missing-cap bottle, so missing-cap ability is unproven |  |
| Augmentation | fliplr 0.5, degrees 0, mosaic, HSV |  |

More epochs? Yes, for YOLOv8s: 200 epochs, patience 50, imgsz 960 (cap and label edges are small at 640), degrees 5, shear 2, fliplr 0 once defect classes like label_skewed exist. Do this after adding production-camera images, or the extra epochs only fit the old camera better.

### 4.3 Segmenter (Stage 3)

Interface only, no weights. Any recipe requiring segmentation FAULTs every bottle. Keep it off until Phase 4.

### 4.4 Decision engine

| Finding | Effect | Fix |
| --- | --- | --- |
| Decision.confidence is computed but never gates PASS | A defect score of 0.70 under a 0.75 threshold passes | Three-zone rule (§6) |
| Frames without a bottle are dropped before the vote | 1 bottle-frame out of 13 can decide | min_bottle_frames_frac (e.g. 0.8) or FAULT |
| check_margin defaults to 0 | No unsure band on the line | Replaced by the three-zone rule |
| Live threshold_shift -0.3 and threshold_overrides | Thresholds lowered under the operator's eye, not in the registry | Remove both. Thresholds come only from the activated model |
| Detector gates lowered to 0.2 in settings | Clutter becomes a "bottle" | Gates come from validation (§6) |

## 5. Training-data audit

### 5.1 What exists

| Class | Images | Scenes | Commercial target (minimum) |
| --- | --- | --- | --- |
| good | 72 | 8 | ≥ 2,000 images from ≥ 200 different bottles |
| damaged_bottle | 319 | 25 | ≥ 400 / ≥ 60 bottles |
| damaged_label | 229 | 21 | ≥ 400 / ≥ 60 |
| skewed_bottle | 167 | 17 | ≥ 300 / ≥ 50 |
| tilt_cap | 145 | 18 | ≥ 400 / ≥ 60 |
| water_level (low / high) | 112 | 12 | ≥ 300 each / ≥ 50, split into low and high |
| skewed_label | 71 | 8 | ≥ 300 / ≥ 50 |
| missing_label | 30 | 2 | ≥ 300 / ≥ 50 |
| missing_cap | 22 | white background | ≥ 300 / ≥ 50, in the production enclosure |
| multi-defect | 0 | — | ≥ 10 % of defect images |

"Scene" = one physical bottle and pose recorded as consecutive frames. The real sample size is the number of scenes, not the number of frames.

### 5.2 Problems

- Wrong camera and geometry. Every training image is 2537×1927 landscape; production is EMEET 1920×1080 rotated to portrait, different lens, distance, light and background.

- Too few independent bottles. ~106 scenes. Good bottles: 8. This is why good bottles are called defective (README: 10 of 19 on the older test set).

- Single-label only. Real defects co-occur. The model has never seen two at once.

- Background shortcut. The missing-cap images are the only ones on a white background.

- No negative clutter. No empty belt, hands, adjacent bottles or partially visible bottles. That is exactly what the line sees between bottles.

- No box-level defect data (§7).

### 5.3 Data collection plan (Phase 1, on the real line)

- Lock the cameras (focus, exposure, white balance; camera_controls) and the lighting before collecting.

- Use the existing Auto-collect (Live page) + Label inbox + hard-examples loop. It is built and self-tested.

- Seeded defect set: make ≥ 50 bottles per defect by hand (loosen or tilt caps, cut or skew labels, dent bottles, under/over-fill) and run each one through the line ≥ 5 times at different rotations.

- Good set: ≥ 200 different production bottles, each run ≥ 5 times.

- Negatives: empty belt, hand in view, two bottles touching, bottle at the frame edge.

- Split by physical bottle id (write it on the cap or base), not by frame or folder. Test set = bottles never seen in training, recorded on a different day.

- Keep the old 2537×1927 data as a pre-training set only. Fine-tune and validate on production-camera data.

## 6. Confidence and threshold design ("very high confidence")

Raising thresholds by itself does not give high confidence. With uncalibrated scores, a higher threshold only turns real defects into passed defects. Industrial systems use a calibrated score and three zones, and treat the middle zone as "not sure, reject":

for each defect d (calibrated probability p_d):
    p_d ≥ reject_min[d]  (e.g. 0.90)           → DEFECT d, confident
    p_d ≤ pass_max[d]    (e.g. 0.05–0.10)      → no d, confident
    otherwise                                   → UNSURE (d)

bottle PASS   only if: every d confident-clear in ≥ 80 % of frames
                       AND detector anchor (bottle) ≥ 0.80 and each required part ≥ 0.70 in those frames
                       AND a bottle was found in ≥ 80 % of the frames
bottle REJECT if:      any d confident-defect by majority vote (defect name + box reported)
everything else        → FAULT "UNSURE" → physically rejected, counted as its own KPI

Implementation (Phase 1):

- Calibrate: fit temperature scaling per output on the (production-camera) validation set and store T in the checkpoint. infer.Model.predict returns sigmoid(logit / T).

- Choose thresholds by target, not by F1: reject_min[d] = the highest threshold that still keeps defect recall ≥ 99.5 % on validation. pass_max[d] = the lowest threshold that keeps good-bottle false alarms ≤ 1 %. Report both on the held-out test set. Fix train.evaluate (max() returns the lowest tied threshold): choose the middle of the perfect band or apply the recall target.

- Best epoch by validation loss, with macro-F1 as the tie-break, not the earliest perfect epoch.

- Remove threshold_shift / threshold_overrides from the production path. Live tuning stays in a "commissioning" mode only and is logged.

- New settings.json keys (defaults shown):

"decision_rules": {
  "vote": "majority",
  "min_bottle_frames_frac": 0.8,
  "pass_max_default": 0.10,
  "reject_min_default": 0.90,
  "det_min_conf_by_class": {"bottle": 0.80, "cap": 0.70, "label": 0.70},
  "unsure_action": "REJECT"
},
"fault_action": "REJECT"

- KPIs on the Production page: good / defect / unsure / fault. An unsure rate > 2 % means the model needs data, not lower thresholds.

Acceptance gates for production (extend registry/model_checks.py):

| Metric | On the production-camera test set (bottles never seen in training) |
| --- | --- |
| Defective bottles passed (escape rate) | ≤ 0.1 % (0 of ≥ 1,000 defect presentations) |
| Good bottles rejected (false reject) | ≤ 1 % |
| Unsure rate | ≤ 2 % |
| Repeatability | same bottle × 50 passes → same verdict ≥ 49 times |
| Per-defect recall | ≥ 99 % for every class with ≥ 100 test presentations |

## 7. Defect classes with localisation

Today the detector finds components (bottle, cap, label) and the classifier gives image-level defect scores. Nothing says where a defect is. The plan adds defect classes to the detector and turns geometric defects into measurements.

### 7.1 New class set (detector v4)

| Class | Kind | Box on | Bootstrapped from |
| --- | --- | --- | --- |
| bottle, cap, label | component (kept) | as today | annotations_v2.json |
| cap_tilted | defect state | the cap | cap boxes on images labelled tilt_cap |
| neck_bare | defect evidence | bare neck / tamper ring | new boxes on missing_cap images (positive evidence, not just "no cap box") |
| label_damaged | defect region | the damaged area of the label | label box on damaged_label images, then tightened by hand |
| label_skewed | defect state | the label | label boxes on skewed_label images |
| bottle_damaged | defect region | the dent / crush / crack | new boxes on damaged_bottle images |
| fill_line | measurement | the meniscus | new thin boxes on every image (good and water_level) |

missing_label stays a recipe rule (no label inside the bottle box), with bottle as evidence. skewed_bottle and tilt_cap also become angle measurements (7.3), with the box class as a second opinion.

### 7.2 Bootstrapping the annotations (no labelling from zero)

- Run the current YOLO on the 1,167 classification images → bottle / cap / label proposals (vision/autoannotate.py already stores proposals separately and never exports them unreviewed).

- For an image whose labels.csv row says tilt_cap, rename its proposed cap box to cap_tilted. The same for skewed_label → label_skewed, and damaged_label → label_damaged.

- Region classes (bottle_damaged, label_damaged tightening, neck_bare, fill_line) are drawn by hand in the Annotate page, most-doubtful first (existing queues).

- A person accepts every image. Accepted boxes go to a new stage2_dataset/annotations_v4.json (v2 stays frozen; it is hash-pinned in MODEL_PROVENANCE.json), a new split_v4.json split by physical bottle, and yolo_export_v4/.

- Then add production-camera images (§5.3) and annotate them the same way.

Estimated effort: ~1,200 images × ~20 s review ≈ 7 h, plus ~600 hand-drawn region boxes ≈ 5 h.

### 7.3 Measurements instead of guesses (industrial standard)

| Defect | Measurement | Rule |
| --- | --- | --- |
| Fill level | fill_line centre y relative to the bottle box, in mm (scale from the known bottle height) | reject outside nominal ± tolerance |
| Cap tilt | cap box angle (YOLOv8-OBB) or two keypoints on the cap rim (YOLOv8-pose) | reject > N° |
| Bottle skew | bottle axis angle from the bottle OBB / keypoints | reject > N° |
| Label skew | label top-edge angle vs bottle axis | reject > N° |
| Label position | label centre vs bottle box | reject outside the zone |

These are explainable to a QA manager ("cap tilted 7.2°, limit 5°") and are tuned in degrees and millimetres, not probabilities. Ultralytics 8.1 supports OBB and pose.

### 7.4 Code touch points

| File | Change |
| --- | --- |
| vision/detect.py | CLASS_NAMES from the model + a per-class kind (component / defect / measurement); the require= check accepts the v4 set |
| line/decision.py | detection_findings: a defect-class box inside the anchor bottle → finding with its box and confidence; measurement rules; recipe defects: section |
| projects/om_bottle/config.json | recipe gains "defects": {"cap_tilted": {"min_conf": 0.8}, ...} and "measurements": {"fill_mm": [x, y], "cap_tilt_deg": 5} |
| line/production_store.py | store defect boxes per bottle (data.defect_boxes) |
| line/machine_cycle.py::_thumb, vision/infer.py overlay | draw the defect box and its name on the evidence image |
| ui/annotation_studio.py, vision/annotate.py | class list from the recipe; v4 export |
| stage2_dataset/ | annotations_v4.py, split_v4.py, export_yolo.py --version v4, validate_yolo_export.py for v4 |
| registry/model_bench.py | yolo --data v4, per-defect-class metrics in the candidate JSON |
| registry/model_checks.py | per-defect-class recall gates |

### 7.5 Does the classifier stay?

Yes, as a second opinion on the bottle crop from the detector (§7.6), for texture defects that boxes describe poorly. A REJECT then names the class and the box when the detector found it, or "classifier: damaged_bottle 0.97" when only the classifier did.

### 7.6 Detector-guided crop (replaces the fixed ROI)

Crop the classifier input from the detected bottle box (plus a margin), not from a fixed ROI. This fixes the landscape-vs-portrait mismatch (§4.1) and any bottle position on the belt. Training uses the same crop function on the training images' bottle boxes. It remains one code path (dataset.model_input gains a box= argument).

## 8. Model archive convention

Request: keep old models in folders named after the data they were trained on.

Proposed layout (nothing has been moved yet; needs your confirmation, see §11):

models_archive/
  2026-10-08_cls-ea2494b8_det-v1v2v3_pre-retrain/     ← today's models_archive/previous, renamed
    ARCHIVE_INFO.json        { date, reason, dataset: {labels_csv_sha256, split, det_annotations, det_split},
                               active_at_archive: {classifier, detector}, models: [...] }
    om_bottle_models/<stamp>/...
    stage2_yolo/...
  2026-10-XX_cls-94289df5_det-v3_pre-v4/                ← the 2026-10-08 candidates, when v4 replaces them

Folder name = <archive date>_cls-<first 8 chars of labels.csv sha256>_det-<detection dataset version>_<reason>. The sha256 is already recorded per checkpoint (test_metrics.json labels_csv_sha256).

Code (Phase 0): tools/archive_models.py:

- --plan: dry run listing what would move.

- --run: moves, writes ARCHIVE_INFO.json, refuses to move the ACTIVE model, appends to models/deployments.jsonl.

- The registry learns to list archived models and to roll back to them (copy back, verify sha256, activate).

## 9. Commercial readiness checklist (beyond the AI)

| Area | Today | Needed |
| --- | --- | --- |
| Cameras | 2 × EMEET webcam, rolling shutter, autofocus, shared USB 2.0 hub | Global-shutter industrial cameras, hardware trigger, fixed lens, one bus each |
| Lighting | room light | Enclosure + back-light (fill, clear-bottle defects) + diffuse front-light (labels, caps) |
| Position | time × speed estimate | Encoder; PLC-side FIFO / shift register |
| Reject confirmation | Y0 observed by polling | Reject-confirm sensor (bottle left the belt) + full-bin sensor |
| Safety | software HALT; no E-stop input in the ladder | Hardwired E-stop category per risk assessment; E-stop status input; Y0 interlock |
| PLC link | Modbus ASCII 9600 | Modbus TCP/RTU 115200 + heartbeat watchdog both ways; PLC rejects if the PC heartbeat stops |
| Runtime | Python 3.8 (end of life 2024-10), desktop app | Python 3.11+, the inspection core as a Windows service that runs without the UI; UPS |
| Models | manual activation | Registry gates of §6, signed checkpoints, challenge-bottle test at every shift start |
| Traceability | SQLite per project | Plus batch/lot, operator login, audit trail of setting changes, nightly backup |
| Validation | software self-tests | IQ / OQ / PQ protocol; repeatability study; seeded-defect challenge set |
| Security | engineer PIN | Windows user accounts, no internet on the line PC, signed updates |

## 10. Phased plan

### Phase 0: make it safe and consistent (1–2 days)

| # | Task | Done when |
| --- | --- | --- |
| 0.1 | fault_action → REJECT; START refuses PASS unless monitor_only | Self-test asserts it |
| 0.2 | Fix the model references: restore 20260919-164511 + stage2_best.pt, or validate and activate 20261008-081732 + bench_yolov8s_v3 through the Models page | Production START checklist shows both models present |
| 0.3 | Remove threshold_shift / threshold_overrides / lowered detector gates from settings.json | Thresholds = the activated model's |
| 0.4 | run.bat: from vision import dataset as D; drop the auto-migrate; one Python version | Launch does not recalibrate |
| 0.5 | Delete the duplicate classes in plc/client.py; fix the data.yaml paths | selfcheck --full passes |
| 0.6 | Remove the 22 white-background missing-cap images from om_bottle (undo batch exists) | labels.csv missing_cap = 0 until production-camera images exist |
| 0.7 | tools/archive_models.py + rename models_archive/previous (§8) | ARCHIVE_INFO.json written; registry lists archived models |

### Phase 1: exact timing and a confident decision (1–2 weeks)

| # | Task | Done when |
| --- | --- | --- |
| 1.1 | line/clock.py (perf_counter) everywhere | Self-tests pass; no time.monotonic() left on the inspection path |
| 1.2 | Bracketed frame and trigger stamps + uncertainty into association | Bottle record stores trigger_uncertainty_ms |
| 1.3 | Fast trigger poll split from status poll | Measured trigger latency p99 < 60 ms on the real PLC |
| 1.4 | AI on a worker pool; RECENT_FRAMES from the window | _due() lateness p99 < 20 ms under load (self-test with a slow fake model) |
| 1.5 | tools/timing_test.py (LED test) and START gate on a recent measurement | Camera and trigger latency known per camera |
| 1.6 | Three-zone decision, min_bottle_frames_frac, unsure KPI | decision self-tests cover each zone |
| 1.7 | Temperature scaling, target-based thresholds, best epoch by val loss (fix train.evaluate) | Thresholds no longer collapse to 0.05 |
| 1.8 | Lock camera controls; collect production-camera data (§5.3) | ≥ 2,000 good + ≥ 300 per defect, split by bottle id |

### Phase 2: throughput and defect localisation (2–4 weeks)

| # | Task | Done when |
| --- | --- | --- |
| 2.1 | Ladder FIFO / shift register; PC answers each bottle at once with its id; window ≤ 0.5 s | 60 bottles/min in the simulator with 0 untriggered bottles |
| 2.2 | Detector v4 classes (§7.1), bootstrapped annotations (§7.2), split_v4 by bottle | validate_yolo_export v4 passes |
| 2.3 | Train YOLOv8s v4: 200 epochs, patience 50, imgsz 960, workers 0 | Per-class val mAP50 ≥ 0.95; test reported once |
| 2.4 | Detector-guided crop for the classifier; retrain on production data (60–100 epochs, early stop on val loss) | Production-camera test meets §6 gates |
| 2.5 | Decision + record + overlay show the defect box and name | Evidence image shows the box; DB stores it |

### Phase 3: industrial hardware (depends on purchasing)

Global-shutter triggered cameras, lighting enclosure, encoder, reject-confirm sensor, E-stop status input, Modbus TCP, heartbeat watchdog in the ladder. Repeat 1.5 and the §6 gates on the new hardware.

### Phase 4: measurements and validation

OBB / pose models for angles and fill level (§7.3); segmentation for label area if still needed; IQ/OQ/PQ; repeatability study; challenge-bottle routine at every shift start; Windows service; Python upgrade.

## 11. Decisions needed from you

- Phase 0.2: restore the old active models (fastest), or activate the 2026-10-08 candidates (B0 classifier + YOLOv8s v3) through the Models page? Both still fail on the production camera until Phase 1.8 data exists.

- Archive naming (§8): is <date>_cls-<labels sha8>_det-<version>_<reason> what you meant by "named as the previous data"? Should the 2026-10-08 candidates move too, or stay until v4 replaces them?

- Hardware budget: Phase 3 (industrial cameras, lighting, encoder) decides whether the commercial targets in §6 are reachable. With webcams the realistic ceiling is a supervised pilot.

- Ladder changes (Phase 2.1): the ladder is yours and is never edited by this repo. Who changes it?