# PLC Communication

The Python <-> PLC foundation: transport, device map, guarded PASS/REJECT commands, trigger handling, fault behaviour and a commissioning window. Everything below that was measured was measured against the ISPSoft DVP-SS2 simulator. No physical PLC has been connected.

Labels used: VERIFIED (observed on this machine, say where) - USER-STATED (given by the user, not re-derivable here) - INFERRED (reasoned or from general Delta knowledge, not tested) - UNKNOWN - REQUIRES PHYSICAL TEST. Test classes are never mixed: FAKE PLC TEST (in-process fake) / SIMULATOR TEST (ISPSoft) / PHYSICAL PLC TEST (none yet).

2026-10-05: final_year.isp (saved 2026-10-04 14:26) and every backup since 2026-10-03 20:24 decode to the same 7 networks as 0.1 below (VERIFIED-FILE, python -m plc.ladder_check). What the ladder must do for this software, what it already does, and the rungs to add: ../hardware/PLC_LADDER_REQUIREMENTS.md. Note: the .isp header length varies between saves (0xAA in the 01:36 file, 264 bytes now); ladder_check searches for it.

## 0. The ladder as actually saved, and what the simulator does (2026-10-03) -- READ THIS FIRST

### 0.1 Current file: saved 2026-10-03 09:06 (VERIFIED from the file; simulator run PENDING)

The project was re-saved at 09:06 (backup final_year_2026-10-3-9-2-10.~bak). Decoded the same way as below:

| Net | Logic |
| --- | --- |
| 1 | X1 -> SET Y1 |
| 2 | X2 -> RST Y1 |
| 3 | X0 (rising edge) -> SET M2 |
| 4 | M0 -> RST M2, RST M0, CNT C0 K9999 |
| 5 | M1 -> TMR T0 K150, RST M2, CNT C1 K9999 |
| 6 | T0 -> TMR T1 K50, OUT Y0 |
| 7 | T1 -> RST M1, RST M2 (new: replaces the RST M1 that net 6 used to do) |

What follows from the file (INFERRED from the logic; reproduced exactly by FakeLadder in plc/test_simulation.py, not yet observed on the simulator):

- The one-scan Y0 flash is fixed. M1 now stays ON until T1 completes, so Y0 is ON for ~T1, then net 7 drops M1, and on the next scan T0, T1 and Y0 reset.

- Presets are still K150 / K50 = 15 s / 5 s at the 100 ms base, not the stated contract K15 / K5 (1.5 s / 0.5 s). Every REJECT therefore holds M1 for ~20 s. Needs the ladder owner's decision; this project does not edit the ladder.

- REJECT acknowledge semantics. M2 drops at once (net 5), but M1 is HELD by design. PLCService now treats "M2 cleared" as the acknowledgement for M1 (address_map.HELD_UNTIL_DONE, CommandResult.cmd_held = True). M1 clearing later means "reject cycle finished". PASS still needs M0 and M2 cleared.

- Triggers are masked during a reject cycle. Net 5 resets M2 on every scan while M1 is ON, so a bottle reaching X0 in that window (T0 + T1 = 2 s with the contract presets, 20 s with the saved ones) raises no trigger. The same is true if X0 rises while M2 is still ON (previous bottle not answered yet). With watch_x0=True, PLCService reads X0 before M2 in every poll and reports each such X0 edge as a BOTTLE_UNTRIGGERED event. machine_cycle records that bottle as FAULT "NOT INSPECTED" instead of losing it silently. This is a ladder limit on continuous handling; only a ladder change removes it (see 0.2).

- Only one reject can be in the PLC at a time (one T0). After a REJECT, the minimum bottle spacing is T0 + T1.

### 0.2 What a ladder change would need (proposal for the ladder owner, NOT implemented)

The software FIFO (machine_cycle.py) already knows when each bottle reaches the reject station. A ladder that removes the masking would separate "inspection answered" from "fire the cylinder":

- Make PASS and REJECT both acknowledge only the trigger (RST M2), not run a long timer on the command bit.

- Let Python send the REJECT at scheduled_reject_time - T0 (already implemented: dispatch_at), with T0 a short, fixed actuator lead time and T1 the pulse. Or keep T0 = travel and add a PLC-side shift register so several rejects can be in flight.

- Never reset M2 from the reject timer rung (net 5 / net 7), so X0 edges during a reject still create triggers.

Until then, machine_cycle works with the ladder as it is and reports every bottle it could not handle.

### 0.3 Earlier file: saved 01:36 / 02:59 (historical; this is what the simulator measurements below used)

Correction: final_year.isp is not encrypted. After a 0xAA-byte header it holds a raw-deflate stream (zlib.decompressobj(-15)) of ISPSoft's text project. Decoded from the file saved 2026-10-03 01:36 (VERIFIED from the file; node types read as 1 = NO contact, 2 = NC contact, 3 = rising-edge contact, 13 = OUT, 15 = SET, 16 = RST):

| Net | Logic |
| --- | --- |
| 1 | X1 -> SET Y1 |
| 2 | X2 -> RST Y1 |
| 3 | X0 (rising edge) -> SET M2 |
| 4 | M0 -> RST M2, RST M0, CNT C0 K9999 |
| 5 | M1 -> TMR T0 K150, RST M2, CNT C1 K9999 |
| 6 | T0 -> TMR T1 K50, OUT Y0, RST M1, RST M2 |

(The project was re-saved 02:59 with identical logic. A second REJECT run caught Y0 ON in exactly 1 of 126 samples, at ~15.0 s, OFF again ~80 ms later, T1 still 0 -- the one-scan flash.)

Simulator behaviour matches this file exactly (VERIFIED on the simulator, one REJECT sampled every ~75 ms for 15 s): M2 clears at once; M1 stays ON while T0 counts 0 -> 150 (15.0 s, 100 ms base); at T0 = 150 M1 and T0 clear together; Y0 was ON in 0 of 187 samples and T1 never left 0; C1 +1. PASS: M0 and M2 clear within ~14-36 ms, C0 +1, Y0 never ON.

Why Y0 never appears (follows from net 6, not from a guess): when T0 completes, the same rung does OUT Y0 and RST M1. M1 is T0's enable (net 5), so on the next scan T0 drops, the rung goes false, Y0 turns OFF and T1 is reset after one scan. Y0 is ON for about one PLC scan; T1 (K50 = 5 s) can never time out. No Y1/X1 interlock exists on Y0.

Mismatch with the stated contract (T0 ~1.5 s, T1 ~0.5 s, Y0 pulse of T1): the earlier backups (final_year_2026-10-2-*.~bak, up to 23:31) contain exactly that design -- net 5 (M1 or T0)... TMR T0 K15, net 6 (T0 or Y0) and not T1 -> TMR T1 K5, OUT Y0. The 01:36 edit replaced it with the table above. The ladder was not changed by this project; this needs the ladder owner's decision. The M0/M1/M2 meanings in section C are confirmed by the file's own device comments; C0/C1 are counters of PASS / REJECT commands.

Software consequence (at the time): PLCService acknowledged a command only when the command bit and M2 were back to 0, so REJECT returned NOT_ACKED (M1 held for 15 s). Superseded by the M1 HELD_UNTIL_DONE rule in 0.1.

## A. Architecture

inspection engine (later)  --PLCLink-->  PLCService  --PLCClient--> Transport --> PLC ladder --> conveyor / sensor / reject
 (PASS/REJECT decision)                   plc/service.py plc/client.py  TcpTransport (sim)        owns all timing and outputs
                                          1 worker thread               SerialTransport: NOT written

| Piece | File | Role |
| --- | --- | --- |
| Framing | plc/protocol.py | Modbus ASCII frames, LRC, exception replies. Pure functions |
| Device map | plc/address_map.py | The single map: addresses, meanings, safety class, write policy, command/trigger bits |
| Client | plc/client.py | Low-level reads, guarded writes, health. Thread-safe per transaction. Application code should not use it directly |
| Service | plc/service.py | The one owner of the link. Worker thread, polling, M2 trigger events, at-most-once PASS/REJECT with acknowledgement, event log |
| Transport | TcpTransport in client.py | Simulator only. The Transport protocol (open/close/send/recv_frame) is what a serial port must implement |
| Tests | plc/test_simulation.py, plc/test_service.py (fake), plc/handshake_test.py (simulator), plc/commissioning.py --selftest | see section H |
| Window | plc/commissioning.py | Commissioning only; not a production dashboard |

The inspection side depends on the PLCLink surface: connect, disconnect, is_connected, read_bit, read_word, read_status, wait_for_trigger, send_pass, send_reject, health_check. No YOLO, camera or image type appears in plc/. Raw write_bit/write_word exist only on PLCClient and are refused for everything except M0/M1.

Old assets: plc file/delta_sim_test.py is a standalone manual keypad script and is not part of the system. It hard-codes M0/M1/M2/Y0/Y1 addresses (matching the current map) but labels M0 "GOOD" / M1 "REJECT"; the current contract (M0 = PASS command, M1 = REJECT command) agrees, but that comment is not independent evidence. plc_modbus_gui.py does not exist. It is superseded by plc/; leave it or delete it, but nothing imports it. There is one PLC implementation.

## B. Simulator configuration (VERIFIED 2026-10-02/03)

| Setting | Value | Evidence |
| --- | --- | --- |
| Simulator | DVP SS2/EC3_8K (DVPSimulator_SS2.exe) | process list; listening on 0.0.0.0:10002 (netstat) |
| COMMGR driver | "Simulaiton SE" (sic), CON_INTERFACE=7 | final_year.ini, DriverInfo.dri |
| Transport / address | TCP 127.0.0.1:10002 | connect succeeds |
| Protocol / station | Modbus ASCII over TCP, station 1 (the simulator answers any station: it does not enforce it) | frames exchanged; station-2 probe answered |
| ISPSoft session | ISPSoft <-> COMMGR session (port 8895) stayed connected while Python talked to port 10002 | netstat ESTABLISHED, no new COMMGR errors |

## C. Device map (single source: plc/address_map.py)

X and Y are octal, M/T/C decimal. Meanings were USER-STATED and are now confirmed by the decoded ladder (section 0). Addressing was VERIFIED by reading the simulator (and M by Delta's RUN relays M1000..M1003 = 1,0,0,1).

| Device | Modbus addr | Type / direction | Meaning | Python access | Safety class | Sim read |
| --- | --- | --- | --- | --- | --- | --- |
| X0 | 0x0400 FC02 | input | photoelectric sensor, bottle at station | read | input | yes |
| X1 | 0x0401 FC02 | input | start button (SET Y1) | read | input | yes |
| X2 | 0x0402 FC02 | input | stop button (RESET Y1) | read | input | yes |
| Y0 | 0x0500 FC01 | output | reject solenoid | read only | never written | yes |
| Y1 | 0x0501 FC01 | output | conveyor motor | read only | never written | yes |
| M2 | 0x0802 FC01 | internal | PLC -> Python trigger: bottle ready | read only | PLC-owned | yes |
| M0 | 0x0800 FC01/FC05 | internal | Python -> PLC PASS command; PLC resets M0 and M2 | write 1 via PLCService | COMMAND | yes |
| M1 | 0x0801 FC01/FC05 | internal | Python -> PLC REJECT command; PLC resets M1 and M2, then T0 -> Y0 -> T1 | write 1 via PLCService | COMMAND | yes |
| T0 | 0x0600 (FC01 contact, FC03 value) | timer | travel delay, intended K15 = 1.5 s (100 ms base) | read only | timer | yes |
| T1 | 0x0601 | timer | reject pulse, intended K5 = 0.5 s | read only | timer | yes |
| C0, C1 | 0x0E00, 0x0E01 (FC03 value) | counter | UNKNOWN role (user reports counter blocks; role not stated; the encrypted ladder cannot be read) | read only | counter | read OK, idle 0 |
| M1000 | 0x0BE8 | special relay | PLC RUN (Delta-defined) | read only | heartbeat | yes |

The previous map listed M10-M12, T3-T5, Y2, Y3 as "in the ladder list". They are not in the current contract and were removed from the map (any valid name still resolves through lookup() as UNKNOWN / read-only). The older Y1 mystery ("ON but not in the list") is resolved: Y1 is the conveyor. .isp/.ini in the working tree are newer than the ones the earlier verification used (modified 2026-10-03 01:36); the contract above is the user's statement about that current ladder. C0/C1 meaning and the T0/T1 timer base are not verifiable from the file; the handshake test measures T0/T1 durations and compares them with the intended values.

## D. PASS handshake (contract; Python side implemented and FAKE-tested; simulator run pending, see K)

- X0 -> ladder sets M2. PLCService sees M2 rising -> Trigger(id); wait_for_trigger() delivers it once.

- Application inspects, calls send_pass(trigger.id).

- Service re-reads M2/M0/M1 and requires M2 = 1, M0 = 0, M1 = 0; marks the trigger answered before sending; writes M0 = 1 once.

- PLC resets M0 and M2 -> service sees both 0 -> ACKED (with write->response and write->ack latency). Y0 must stay OFF.

## E. REJECT handshake

Same as D with M1, except the acknowledgement is M2 cleared (the 09:06 ladder holds M1 ON through T0 + T1; see 0.1, HELD_UNTIL_DONE). After the ack the PLC runs T0 -> Y0 ON -> T1 -> Y0 OFF -> M1 OFF. Python never times or drives Y0. The ack means "the PLC consumed the command", not "the bottle was ejected"; machine_cycle watches Y0 ON/OFF (status poll, ~150 ms resolution) and records a FAULT if Y0 is never seen for a REJECT.

## F. Timing measurements

- Read latency, SIMULATOR ONLY (N = 200, 2026-10-03): single/block/input/word reads median ~16-17 ms, mean ~19-20 ms, p95 ~32 ms, p99 ~33-41 ms, max 106 ms, 0 failures; read_status() (now 6 transactions) mean 120 ms; connect+heartbeat+ disconnect mean 25 ms; wrong port -> PLCConnectionError/FAULT after ~2.0 s (Windows retries a refused loopback connect). (Yesterday's run: ~12-13 ms, read_status 64 ms. Simulator pacing varies with machine load.)

- Write request -> response, write -> PLC ack, command -> Y0 ON, Y0 ON -> OFF: python -m plc.handshake_test --real prints these (N, mean, median, min, max, p95, p99, failures). NOT YET MEASURED on the simulator (see K).

- None of this is real-PLC or machine timing.

## G. Failure behaviour

| Case | Behaviour | Test class |
| --- | --- | --- |
| wrong port / simulator down | PLCConnectionError, link FAULT, is_connected() False, FAULT event | FAKE + SIMULATOR |
| timeout, garbage, bad LRC, wrong station, peer close | PLCError, FAULT, socket closed, FAULT event, snapshot withheld | FAKE |
| command / read during FAULT or DISCONNECTED | refused before any byte is sent | FAKE + SIMULATOR |
| link lost while M2 active | trigger -> LOST; after explicit connect() M2 seen again is a new trigger flagged after_reconnect | FAKE |
| write sent, reply lost | WRITE_FAILED, FAULT, not retried, unresolved_command flagged | FAKE |
| written but PLC never consumes it | NOT_ACKED, not retried, flagged | FAKE |
| written, link dies before ack | ACK_LOST, not retried | by construction (same path), not separately tested |
| second answer to same trigger / 8 racing threads | exactly 1 write on the wire | FAKE |
| trigger not answered in trigger_overdue_s | TRIGGER_OVERDUE event; not dropped | FAKE |
| no good reply for stale_after_s | link state DEGRADED (never "connected") | FAKE |
| 20 reconnects, shutdown while connected | clean, no writes | SIMULATOR |

There is no automatic reconnect and no automatic retry. A reconnect never replays a command. After WRITE_FAILED / NOT_ACKED / ACK_LOST the application must read the PLC and decide; the service will not decide for it.

## H. Tests (what exists and how to run)

python -m plc.test_simulation                       # FAKE: protocol/addressing/write policy/FAULTs (unit)
python -m plc.test_service                          # FAKE: service + FakeLadder (scan emulation of the 09:06 ladder), 13 groups
python -m plc.commissioning --selftest              # FAKE: HMI against fake PLC
python -m plc.test_simulation --real --bench 200 --faults   # SIMULATOR, read-only
python -m plc.handshake_test --fake                 # FAKE: the harness itself
python -m plc.handshake_test --real --cycles PASS,REJECT,PASS,REJECT,REJECT,PASS --wait 60   # SIMULATOR, writes M0/M1
python -m plc.handshake_test --real --sim-x0 --t0 1.5 --t1 0.5 --csv trace.csv   # ...X0 pulsed by Python, full trace
python -m line.machine_cycle                             # FAKE: the whole per-bottle cycle against FakeLadder

FakeLadder runs the seven decoded nets in order every scan, commits each scan atomically, and serves X from a scan-latched input image, as a real PLC answers Modbus between scans. Three test flakes during development were mid-scan reads in an earlier fake, not service bugs. It is still a FAKE: real scan time, serial timing and the real timer base are only measured by --real.

## I. Simulator limitations

- The simulator answers any station and has no serial timing, no framing errors, no cable faults.

- Trigger stimulus: PLCService.simulator_test_write may pulse X0 (and X1/X2/M0/M1/M2) only on a loopback simulator link, one bit per write, logged as SIM_TEST_WRITE. handshake_test --sim-x0 and the Production tab's "Simulate bottle (X0)" use it. On a serial (real PLC) link it is refused.

- If a second bottle arrives while M2 is still 1, or while M1 is ON, M2 shows no new edge. With watch_x0=True this is now reported (BOTTLE_UNTRIGGERED), not silent, but the bottle is still not inspected (0.1).

- Polling is ~20-50 ms with ~16 ms transactions: a trigger is noticed up to ~one poll period late.

## J. Physical PLC transition and COM-port ownership

Known: a DVP14SS2 family PLC (user: DVP14SS2-1TRW; the usual catalogue numbers are DVP14SS211R/T, so the exact variant suffix is UNKNOWN), a Delta cable the user owns (type UNKNOWN), Modbus ASCII with LRC works against the simulator. INFERRED from general Delta documentation, not tested here: SS2 CPUs have an RS-232 programming port (COM1) and an RS-485 port (COM2, terminal block), both support Modbus ASCII/RTU, factory default is ASCII 7E1 9600 baud station 1 on COM1, and the 0x0400/0500/0600/0800 base addresses match this series. UNKNOWN / REQUIRES PHYSICAL TEST: COM number, baud, parity, data/stop bits, ASCII vs RTU, station, USB-serial driver, whether the cable is RS-232 or RS-485, and whether the PLC's COM1 can be used while a programming cable is also needed.

Ownership (the real constraint): a serial port is opened by one process at a time. While ISPSoft is online through COMMGR, COMMGR holds the COM port and Python cannot open it (and vice versa). Therefore at machine runtime either (1) ISPSoft/COMMGR is closed and Python owns the port, or (2) a second physical port is used (RS-485 COM2 via a USB-RS485 adapter for Python, programming cable stays on COM1). Option (2) is the recommended arrangement if the ladder is still being edited; it REQUIRES PHYSICAL TEST. The application must acquire the port on connect(), release it on stop() (already true for Transport.close()), and tell the operator clearly when the port is busy (an open failure becomes PLCConnectionError -> FAULT, which is correct). SerialTransport (in plc/client.py, pyserial) now exists: Modbus ASCII over a COM port with selectable port, baud and format (7E1/7O1/7N2/8N1/8E1/8O1). It is NOT TESTED ON HARDWARE -- only its framing is tested, on pyserial's loop:// echo port (FAKE). The port/baud/format/station must be read from the real PLC, not assumed (Delta default 9600 7E1 station 1 is INFERRED). PLCService.reconfigure() switches the one service between the simulator and the serial link; the main application's Machine tab has the selector (Simulator TCP / Real PLC serial), Connect, Disconnect and a read-only "Test link" (10 heartbeats). Auto-connect happens only in simulator mode; a COM port is opened only on Connect. Simulator test switches and simulator_test_write refuse to work on a serial link. Modbus RTU is not implemented.

## K. Status of the simulator write path for the 09:06 ladder: NOT YET RUN

The 01:36 ladder was exercised on the simulator (0.3). The 09:06 ladder has not been: on 2026-10-03 ~11:00 the DVPSimulator_SS2 was started standalone, but it was in STOP with no program (M1000 = 0), and only ISPSoft can download the ladder. handshake_test --real now refuses to start in STOP and says so. Exact next steps:

- ISPSoft: open final_year, set T0 = K15 and T1 = K5 if the 1.5 s / 0.5 s contract stands (the saved file has K150 / K50), compile, Simulation ON, download, RUN.

- python -m plc.handshake_test --real --sim-x0 --cycles PASS --csv single_pass.csv

- ... --cycles REJECT --csv single_reject.csv

- ... --cycles PASS,REJECT,PASS,REJECT,REJECT,PASS --csv sequence.csv (the harness waits for each reject cycle to finish before the next X0, because a trigger during M1 would be masked). If the presets stay K150/K50, add --t0 15 --t1 5 to check against them instead.

- Freeze the contract (section C + 0.1) once those pass, then run the Production tab against the simulator (Simulate bottle / auto-feed).

## L. Known unknowns / next hardware verification

- Simulator handshake result (above) and the observed T0/T1 against 1.5 s / 0.5 s.

- C0/C1 role (and whether the ladder uses them to count bottles/rejects).

- Physical: COM port, serial settings, ASCII vs RTU, station, cable pinout, port ownership with ISPSoft.

- Physical: real read latency; real X0 -> M2 behaviour with debounce/bounce; M2 chatter.

- Heartbeat/watchdog in the ladder (stale PC -> safe state without PC involvement). Not present in the stated contract.

- What the ladder does if M0 and M1 are both set, or either is set with M2 = 0 (the service never does either).